VOLUME 3

TECHNICAL SPECIFICATIONS

# FOR

# „CONSTRUCTIONAL WORKS IN PALILULA – RECONSTRUCTION OF AMPHITHEATRE IN CITY PARK”

1. PROJECT DESCRIPTION

# GENERAL SCOPE OF THE WORKS

The scope of Contractor’s work covers the following construction works elements: Constructional works in Palilula – reconstruction of Amphitheatre in city park“.

Subject and estimate of works covered by all the work on the arrangement, ground clearance and adapted the old contents of the plateau, an adaptation of the tap, leveling the ground surface, installation of park benches, installation of piles of wooden seats in the auditorium or Amphitheatre, and replacement of concrete basis with seats for spectators.

Papers relating to clearing the ground, creating footpaths that line the curbs and gravel of granite cubes. Parts that are damaged previous works, to enhance by planting seedlings, sow grass surfaces and their subsequent care.

The whole eastern part of the memorial park "Drums" and provided the organization, or repair the amphitheatre which is of great importance for various cultural events and performances, plateaus with benches for rest and recreation, maintenance of school hours in nature and recreation.

In addition to all these works are envisaged rehabilitation and conservation and restoration works on the pyramidal monument of dressed stone.

The implementation of the contract shall be executed with full compliance with the description of the main project which is an integral part of tender documentation. The main project includes explanatory notes, bill of quantities and design drawings.

The Technical Specification is an integral part of the Tender Documentation /TD/ together with the provisions of the Contract, the detailed design drawings and the other contract documents. The Specification specifies and further develops the requirements for the implementation of the construction works under the Contract. In case of ambiguities in the Bill of Quantity /BoQ/ or contradictions between the Specification and the BoQ, the Specification shall prevail.

  - Project Location

Memorial Park `Bubanj` is stands for public surface of great cultural-historical importance for Nis, situated in Nis, at Palilula municipal territory, 2km from the City center. The complex is consists from amphitheater, plateau of memories, and memorial cross “Abdipasina tabija” original monument of executed patriots three fountains, small stage and trail in honor of fallen patriots in 2nd World War. The complex is arranged park area; a memorial trail is approximately half kilometers; a marble relief in the size 23x2.5m consists of five compositions that symbolizing “killing machine”, hanging and shooting, the people revolt, the founder of capitulation and victory, and three concrete obelisks that symbolize uplifted arms with clenched fists.

The total surface area of detailed plan is 42 hectares. Road connection is good. Regarding environmental issue, this area in not polluted since there is no significant industrial facility near the site area. This Memorial Park is edged and located in the area between `Trebinjska` street and magisterial road Nis-Skopje. Location itself  presents popular picnic area, but at the same time very important cultural-historical monument, dated from the after 2nd World War, when it get the function of memorial park with three fists concrete sculptures. Every of the three fists are different size, and they symbolize man, women and kids hands that defy the enemy. From the year 1979, Memorial Park “Bubanj” is cultural property of great importance based on Decision no. 29 of Institute of Monument and Culture Protection of Serbia.

The current state of this area, that functions as park zone and pedestrian area, is not satisfactory and for this matter it should take the measures to refine and improve this complex of city land and become closer in that way to the great number of visitors.

The objective of this project is better appearance of `Bubanj` Memorial Park, and in this way of the city itself, together with preservation of cultural-historical heritage.

The terrain, where is a part of `Bubanj` complex is situated, part for which we have intention to adapt, is mostly in fall, and provided activities for the reconstruction of this area are in the accordance with `Detailed urbanity plan` from `88 and changes from 89 and `92.

With pre-measurement and works estimation, all the works are taken into account regarding adaptation, terrain cleaning and adaptation of current parts, adaptation of plateau, adaptation of fountain, leveling of land surfaces, placing park benches, embedding wooden seats of talp into amphitheater, as well as the replacement of concrete footer of seats in the amphitheater.

Works relate to terrain cleaning, pedestrian paths that are covered with gravel and roadsides of granite blocks. Parts that are damaged with previous works should be improved-refined with seedlings planting, seeding down grass surfaces and their further care.

For the whole eastern part of Memorial Park “Bubanj” it is estimated to be adapted, i.e. amphitheater repairing that is very important location for various cultural manifestations and events organization, plateaus with benches for resting and recreation, organization of school lessons in nature and other ways of entertainment.

Beside all these works, sanitation and conservatory-restoration works on the body of pyramid monument of hewed stone are all also estimated.  All the works will be done according to phases of clearly defined dynamics.

All works will be performed under the supervision of the `Institute for monument protection` – authorized institution for cultural monuments state. On the basis of `Law on planning and construction` due to Article 146 it is not necessary to get special structuring license for these kinds of actions.

The conclusion comes itself that this project objective is creating ambience, and providing more quality visit to visitors of `Bubanj` and the citizens.

General Technical Specifications

# FOR

# „CONSTRUCTIONAL WORKS IN PALILULA – RECONSTRUCTION OF AMPHITHEATRE IN CITY PARK”

# 0. GENERAL SPECIFICATIONS

# PART 1 – GENERAL

    - Standards:    As a minimum the Serbian standards and codes shall always be satisfied where applicable. Other internationally acknowledged standards and codes may be used only if: They are more or at least equally stringent compared to the respective Serbian standards and codes or  no applicable Serbian standards and codes exist for the specific case.

    - Units of Measurements: In general, the units of measurements to be used in connection with this contract are metric units of mm, cm, m Km, N (Newton), Mg (1000 kg) and degrees Celsius (Co). Decimal points are written as “,  “.

    - Programmed for the Execution of the works: The Contractor shall submit to the Supervisor  a fully detailed programme showing the order, the procedure and method by which he proposes to carry out the construction and completion of the Works.

    - Information to be Supplied to the Supervisor : The information to be supplied to the Supervisor  shall include drawings showing the general arrangement of the temporary offices and any other temporary buildings or structures which he proposes to use, together with details of the constructional plant and temporary works, and all other devices which he proposes to adopt for the construction and completion of the whole of the works and, in addition, details of the labour strength, skilled and unskilled, and supervision arrangements.

    - Approval by Supervisor  of Method Statements: The manner and the order in which it is proposed to execute the permanent works as described in the Contractor’s method statements is subject to adjustment and approval by the Supervisor , and the Contract price shall be held to include any necessary adjustment required by the Supervisor  during the course of the work.

    - Approval of sources, materials and plants: Materials and plants incorporated must originate in a Member State of the European Union or a country covered by the Bulgaria-Serbia IPA Cross-border programme.Material that originates from sources, which are not approved by the Supervisor, cannot be used for Works. Approval of a source does not mean that all material in the source is approved. The Contractor has to ascertain by continuous control check measurements that only material which complies with the requirements specified in the various clauses of these specifications will be used for the Works. If the supervisory authority is requested to examine some of the material, contractor should at his own expense, conduct it through a well-known institutes and to submits a proper certificate to the supervisory authority. If a certificate proving that the material is irregular, it will be immediately cleared from the site. Otherwise the contractor has the right to request an extension of time If the supervisor authority believes that a certain built-in material is impropriate or that some of the works are carried out in non-adequate manner, it should order to contractor through construction log demolition and destruction of the scope of works and removal of materials from the site. Supervisor work gives the reason in construction log, so that contractor could later complain if these works were not appropriate. Contractor is responsible for the quality of the total installed material and for the material left at the disposal by the investor. If the Contracting Authority believes that the contractor’s material is not the proper quality, he will refuse to be installed and this will conclude the construction log. Only with the injunction of the supervisory body via construction log, he will install the material, where he will be no longer responsible for it and for the consequences arising from the application of the same.

    - Faulty works: Any work, which fails to comply with these Specifications, shall be rejected and the Contractor shall, at his own expense, repair any defects, as required by Supervisor.

    - Advertising, Boarding, etc.: No advertising material whatever will be permitted to be displayed on the site except that:

The contractor shall construct one information boards erected in the highly visible locations. The wording shall be presented in such a way as to be legible at a distance of 100 meters. The Contractor shall manufacture two (2) sign/notice boards and erect these where directed by the Engineer within the working limits. The notice boards shall be 1.00 x 2.00 m of 20 mm thick plywood or similar approved with softwood surround. The notice boards shall have the indication as instructed by the Engineer and be painted after fabrication on all surfaces. The Contractor shall erect these boards on suitable supports in positions within the Site as directed by the Engineer. The Contractor shall allow for installation and maintain in good condition It is necessary to provide the visibility of the project and to provide on board the name of the project and the co-financing institutions written in a way /text and font size/ conforming to the requirements of the Contract and in compliance with EU visibility guidelines and the guidelines by the Bulgaria-Serbia IPA Cross-border programme. The cost of these boards will be included in the Contractor’s rate in Bill of Quantities.

    - Submissions to the Supervisor : Written Authority: “Order in writing” shall mean any document or letter signed by the Supervisor or his representative  and posted or delivered to the contractor and containing instructions, guidance or directions to the contractor for the execution of the Contract.  Whenever the word approved, directed, authorized, required, permitted, ordered, instructed, designated, considered, necessary, prescribed, or words (including nouns, verbs, adjectives and adverbs) of like important are used, it shall be understood that the written approval, direction, authority, requirement, permission, order, instruction, designation, prescription, etc. of the Supervisor  is implied unless another meaning is plainly intended.

The contractor should submit to the Supervisor or his representative every additional work; a detailed drawing and the work should begin only after Supervisor approval. All the cost occurred will be covered by Contractor.  The contractor should sign proposals, details, sketches, accounts, information, materials, test certificate, whenever requires by Supervisor or his representative. The Supervisor will accept every submission and if appropriate will be answered to the contractor in accordance to any proper clause of contract conditions. Every submission should be done in dates as agreed with Supervisor or his representative.

    - Samples: The contractor should provide samples, labelled due to all fittings, accessories, and other issues might be asked from the Supervisor or his representative for inspection.  The samples should be submitted to the Supervisor’s office.

    - As Built Drawings: The drawings of implemented works and measurement hand-book.  The contractor will prepare and submit to the Supervisor or his representative three sets of As-Built documentation. This material should contain the complete set of drawings of the implemented works, including any additional drawings made during work implementation approved by the Supervisor, and the measurement handbook per each work volume.

    - Road cleaning: Streets adjacent to the site shall be kept clean on a daily basis. The Contractor shall take necessary precautions to prevent excessive spoiling of streets such as wheel washing and covering of open trucks and containers leaving the site.

    - Radios on Site: The use of radios and the like on site is prohibited so that noise pollution is kept to a minimum.

    - Protection of Trees: All trees which are not approved for felling shall be protected against damage during the entire Contract Period using wooden boards strapped around the truck of the tree extending up to a height of 2m. No trees with a truck diameter greater than 10 cm measured at 1m above ground shall be felled without prior permission of the relevant authorities and the Supervisor.

    - Health and Safety: The Contractor shall implement all works in accordance with current European and Serbian Health and Safety at Work Standards. He shall develop and maintain a proactive approach to safety on site with a site safety plan, nomination of a safety inspector, carrying out regular staff safety briefings, performing regular safety tours of the site with the Supervisor or his representative, maintaining of accident statistics for presentation to the Supervisor or his representative each month etc

1.1.15.	Implementation programme for delivery of works shall be prepared consistent with the duration of the project’s implementation and the works activities implementation. The programme shall be presented in a format which includes a bar chart of all the major activities. No later than five working days after the end of each month, the Contractor shall submit an updated programme to the Supervisor. This shall show, as a minimum:

- progress made during the past month, compared with planned progress

- activities planned for the next month.

The General project, written in Serbian, is part of the contract between the investor and the performers. Destroying the plans from each side of the party is considered to be avoiding the obligations and responsibilities, where the true statement and plans will be take in consideration from the opposing sides. Plans must be signed by both parties. Structural drawings shall be read and used in conjunction with relevant Architectural drawings. All Structural materials used shall be in conformation with relevant Serbian Standard Codes. Dimensions mentioned in structural drawings will be in millimetre unless otherwise mentioned. Before the start of work, Contractor need to compare plans with actual state of the buildings and report investor on all disagreements. If this step is not taken by contractor, the contractor takes the risk for subsequent work as a result of disagreement, unless the discrepancies are errors and omissions of designer of General Project. The agreed price shall be calculated to set off entire work, tools and materials for installation, as well as the entire transport, wages, social charges, etc. The contracted price should include three copies of technically developed complete plans of performed installation, as well as three technical guidelines for handling the installation which should be visible and hung on a suitable place to be able to use by the operating personnel. In agreement with the contractor should be indicated a contact person who manages the work and has the legal right to work on this position. Also in the contract should be indicated a person who will present on-site supervision and carry out its function of all the works on site, which has a legal right to this position. In contract with the contractor to predict the manner and timing for the preparation, filing and payment of situation, final costing, the date of execution of works, the guarantee period, penalties for both parties, etc.

1.1.16. Contractor's equipment-The Contractor shall furnish equipment which will be efficient and appropriate to secure a satisfactory quality of work and a rate of progress which will insure the completion of the Works within the time stipulated in the Tender. If at any time such equipment appears to be inefficient, inappropriate or insufficient for securing the quality of work required or for the rate of progress, the Supervisor may be entitled to order the Contractor to increase the efficiency, change the character or hire additional equipment, and the Contractor shall conform to such order.

1.1.1.17 Protection of existing structures and utilities- The Contractor shall assume full responsibility for the protection of all buildings, structures and roads existing in the area of the construction site, public or private, whether or not they are shown on the drawings.  The Contractor has to pay special attention to avoid any damages on any protected areas. Damage caused by force majeure repair contractor at his own expense, but this gives him the right to extend the deadline.

1.1.1.18. Electrical installations- Based on technical conditions for electrical works delivered by “Elektrodistribucija”, number 73432/1, from 03.09.2010 it is necessary to comply with the following:-walls and foundations need to be in range more than 0.3m. It is necessary for Contractor  to make an appointment in Sector of maintains EEO of “Elektrodistribucija”, regarding precise tracing of available cables already on sites, as well as per moving installed cables which could be damaged, or in condition that not satisfy Technical Conditions. All cost for moving of cables should be fall on expense of Contractor. Contractor is obliged to inform Sector of maintains EEO of “Elektrodistribucija”  five days before starting of work activities, as per common checking of traces of existing energy cables. It is necessary to provide to above mentioned authority:

-technical condition for project

-situation of existing electro-energetic objects

Works near existing electro-energetic objects is need to be handmade, in agreement with authorized person from the above mentioned authority. All electrical power required by the Contractor shall be provided by him at his own expense. The Contractor prior to taking-over of the Works shall remove all temporary installations if it is not agreed upon that the Contracting Authority takes over the installations. Any damage resulting from the Contractor's operations shall be repaired at his expense.

1.1.1.19. TT Objects- Contractor is obliged to provide TT works in accordance with existing   technical conditions.  Contractor is obliged to the following:

- Contractor is obliged to predict full protection, or moving of underground TT objects if they are exposed to possible damage, at his own expense

- If it comes to crossing or parallel installment other underground installation with telecommunication installations, it is necessary to provide minimum requirements as pre follows:

| Sort of underground or ground object | Horizontal distance (m) | Vertical distance (m) |
| --- | --- | --- |
| gas with middle and low pressure | 0.4 | 0.4 |
| Energetic cable up to 10KV | 0.5 | 0.5 |
| Energetic cable above 10KV | 1 | 0.6 |
| Water supply installations | 3 | 0.3 |
| Sewage installations | 0.6 | 0.5 |

Contractor is obliged to inform Telokom Srbija in written form about time of starting of works, in order to agree on TT cables tracing on the site. Contractor is obliged to provide presents of technical person of Telekom Srbija before starting of works on traces of crossing or parallel tracing with traces of TT objects. Ground Works near existing TT objects is need to be handmade, in agreement with authorized person from the above mentioned authority and with the presence of technical person of the above mentioned authority. In case of unpredicted circumstances of vulnerability of TT objects, contractor is obliged to immediately stop the work and inform above mentioned authority, in order to prepare record on damaged on TT objects. All TT required by the Contractor shall be provided by him at his own expense.  Any damage resulting from the Contractor's operations shall be repaired at his expenses

1.1.1.20. Water and sanitary-Directly on site, it is necessary to provide minimum distance from public supply of water at minimum 3m, and from an enclosed reservoir area near the street Trebinjsa, minimum 10m.

Existing street caps and underground hydrants should be placed in the level of the plateau and slopes.

Before starting of works on site, it is necessary to get permission from JKP Naissus, Office for systematic analyse NIVOS-a and NIKAS-a, visible signs for existing public traces. It is necessary to prepare site record and schemes from person authorized by Contractor.

For connection of planned objects to public facilities, it is necessary contact above mentioned PUC. Water supply network on the site is I and II highlands of water supply. Technical solution and dimensions of water meter manhole is necessary to obtain from PUC Naissus , after their site visiting. Contractor is obliged to deliver project of installation of objects planned for connection to network.

1.1.1.21. Safety and security on site- Safety and security arrangement should be performed in accordance with article Serbian Law on planning and construction and related regulations.

1.1.1.22. Handling and storage of materials and plants-All materials and plants to be incorporated in the work shall be handled and stored in a manner, which prevents injury of any kind whatsoever. Any materials or plants which, in the opinion of the Supervisor, have become too damaged to be fit for the use intended or specified shall be promptly removed from the site, and the Contractor shall receive no compensation for the damaged material or its removal.

1.1.1.23. No specific provision- Where no specific provision is made in the Bill of Quantities for work of a temporary nature, it shall be considered as covered by the unit prices entered in the Bill of Quantities.

1.1.1.24. Clean-up work-The Contractor shall clear way and remove from the site any wreckage, rubbish and temporary works, which are no longer required.

# 1. DEMOLITION AND DISPOSAL

# PART 1 - GENERAL

  - REFERENCES: The publications listed below form a part of this specification to the extent referenced.  The publications are referred to in the text by the basic designation only.

    - Applicable Serbian Laws and Norms:

    - European Norms and Standards:

DIN 18007	Demolition work - Terminology, techniques and applications

DIN EN 74	Couplers, loose spigots and base-plates for use in working scaffolds and false work made of steel tubes; requirements and test procedures.

DIN IEC 60364-7-704	Erection of low-voltage installations - Part 7-704: Requirements for special installations or locations - Construction and demolition site installations (IEC 64/1339/CD:2003)

  - SUBMITTALS: Submit the following:

    - Statements: Submit proposed demolition and removal procedures to the Supervisor or his representative for approval before work is started. Procedures shall provide for careful removal and disposition of materials and coordination with other works in progress. The Contractor shall carry out at his expense a detailed condition survey, a disconnection schedule of utility services, all necessary permits from utility supply companies, a detailed description of methods and equipment to be used for each operation and of sequence of operations.

    - Condition Surveys: In cases where neighbouring property could be damaged by the Works then the Contractor shall carry out at his expense a detailed condition survey of the neighbouring property including photographic documentation a copy of which shall be presented to and approved by the Supervisor or his representative prior to commencement of the works.

  - REQUIREMENTS: The work includes demolition and removal of all items indicated or specified. Demolition works shall not be commenced until authorisation is received from the Supervisor or his representative. All the material resulting from demolition work, except as indicated or specified otherwise, shall become the property of the Contractor and shall be removed only with permission of the contractor’s representative from the construction site to the disposal site indicated by the Municipality. Rubbish and debris shall be removed from the site daily, unless otherwise advised by the Supervisor or his representative; the accumulation of such material is prohibited both inside and outside the site. Materials which cannot be removed form the site daily shall be appropriately stored in areas designated by the Supervisor or his representative. The Contractor shall provide suitable containers for the disposal of miscellaneous construction debris and shall empty these as often as required. All containers shall be covered at all times so as to prevent dust and debris being lifted by wind. Should the Contractor identify dry rot in wood or asbestos during demolition works he shall take appropriate health and safety precautions, notify the Supervisor or his representative immediately and propose suitable disposal measures for approval by the Supervisor or his representative. The Contractor shall produce evidence of the proper disposal of materials upon request of the Supervisor or his representative.

# PART 2 - EXECUTION

  - EXISTING FACILITIES:

2.1.1	Utilities and Related Equipment: Carefully remove existing utilities uncovered by the work as indicated and terminate in a manner conforming to European norms/regulations covering the specific utility and as approved by the Supervisor or his representative. If utility lines are encountered that are not shown on the drawings, contact the Supervisor or his representative for further instructions.

    - Under ground Utilities: Prior to commencing demolition works the utilities in the demolition area shall be taken out of service and secured so that unintentional reinstatement is prevented.

    - Concrete: Saw cut concrete along straight lines to the point where the road intersect the other road

    - Walls: During demolition of walls and/or modifications to openings in walls care shall be taken to ensure clean cut finishes at the extents of demolished sections. Care shall be taken not to subject floors from excessive impact loading arising from falling rumble.

  - EXCAVATIONS:

Cover or fill all excavations, open trenches and pits, and other hazardous ground openings.

  - PATCHING:

Wherever existing surfaces of permanent works are damaged by the Contractor during the performance of the work, the Contractor shall repair, patch, and finish such surface(s) to match finish of adjacent undamaged surface(s).

  - DISPOSITION AND TITLE TO MATERIALS:

2.4.1	Disposal of Existing Materials and Equipment: Except where indicated or specified otherwise in other sections, all materials and equipment removed, and not reused, shall become property of the Contractor and shall be removed from the Site. Title to all materials resulting from demolition, and all materials and equipment to be removed, is vested in the Contractor upon approval by the Supervisor or his representative of the Contractor's demolition and removal procedures, and upon authorisation by the Supervisor or his representative to commence demolition works.

The Client will not be responsible for the condition or loss of, or damage to, such property after Notice to Commence. Materials and equipment shall not be viewed by prospective purchasers or sold on the site.

  - NOISE & VIBRATION CONTROL: In cases where demolition shall take place in buildings still in operation; noise & vibration intensive works shall not be permitted during normal working hours so that disturbance to operation is kept to a minimum. Coordination of such works shall be made with the Supervisor or his representative.

  - DUST CONTROL: Take appropriate action to check the spread of dust and avoid the creation of a nuisance in the surrounding area. Comply with all dust regulations imposed by the site and local air pollution agencies. During demolition in buildings which are still in use dust curtains shall be provided to protect those areas still under operation from air borne dust. When demolishing elements containing fibre materials then care shall be taken to avoid displacement of fibres and thus inhalation. Moistening of such material and the use of dust masks shall be deemed a minimum protection measure. Drilling into fibrous material is forbidden.

  - PROTECTION:

    - Existing Work: Before commencement of the works the Contractor shall together with the Supervisor or his representative inspect and identify all existing items which shall be kept or reused. Such existing work which is: (a) to remain in place, (b), to be reused, or (c) to remain the property of the Client, shall be protected using temporary covers, shoring, bracing, and supports. Items which are to remain and which are damaged during performance of the work shall be repaired to their original condition or replace with new at the expense of the Contractor. Do not overload structural elements. Provide new supports and reinforcement for existing construction weakened by demolition works. Should loss of structural stability be detected (excessive deflections, collapse etc) then the Contractor shall take immediate action to secure the works and evacuate and seal off the area and inform the Supervisor or his representative.

    - Lowering of Materials: When materials or debris are lowered from heights, care shall be taken to prevent the material swinging, falling or being projected in such a manner that it creates a danger to the safety of personnel or public property of any kind.

    - Debris Chutes: Should the Contractor utilise debris tube chutes to transfer debris from upper floors of a building into containers then these chutes shall be adequately restrained and installed by an experienced fitter and shall be air tight at joints. The receiving container shall be covered with suitable sheeting which in turn shall be sealed around the slide as it enters the container so as to prevent excessive dust rising out of the container. Suitable and secure access shall be provided at the top of the slide to ensure safe working conditions when depositing debris into the chute.

    - Existing Services: Existing electrical wires and conduits that are to remain shall be carefully detached from their anchors/ fasteners (without disconnection), repositioned out of the way of current construction operations, and protected from damage. Prior to demolition of works the Contractor shall check for the location of concealed services.

    - Winter Damage: The materials on site which can be damaged shall be protected from winter damages such as freezing of water pipes etc during demolition works and construction work.

    - Replacement of Windows and Doors: During the replacement of windows and doors the Contractor shall ensure that any existing items removed are replaced on the same day so that the building is secure overnight (where it is required).

    - Roofing Works: During roofing works the Contractor shall ensure the building remains weather tight at all times. Temporary roofing material such as weather proof sheeting shall be provided as and when required. Generally the removal of roofing shall be done in stages coordination with the roofing works so that the requirement of temporary roofing is kept to a minimum.

    - Shut down Periods: During periods of non attendance on site the Contractor shall ensure that all permanent and temporary works are secure and shall visit the site on a daily basis to check the condition of the Works.

  - HEALTH & SAFETY:

    - Experienced Supervision: All demolition and construction work shall be under constant supervision from experienced and qualified Supervisor or foreman.

    - Protection of Pedestrians: Where pedestrian and driver safety is endangered by the demolition work, use traffic barricades with flashing lights (during hours of darkness) and appropriate signs to redirect traffic away from the demolition area. Provide necessary temporary false work and covers to shield passers-by.

    - Hot Working: The storage of pressure gas cylinders in basements, stairwells, corridors and escape routes is forbidden. During hot working with inflammable gases hand held fire extinguishers according to DIN EN 3 must be readily available immediately adjacent to the area of working.

    - Safety Clothing: During demolition works all workers shall wear proper safety clothing including but not limited to eye protection, dust masks, safety shoes, heavy working cloves and hard hats.

    - Metal Cutting: During cutting works using grinder or blow torch methods especially in service shafts the Contactor shall ensure that sparks and molten metal due not fall into uncontrolled areas.

  - SCAFFOLDING:

All scaffolding shall be designed and erected in accordance with the relevant standards. Only experienced and competent scaffolding erectors shall carry out erection. The Contractor shall ensure that any necessary modifications to the scaffolding during the course of the works shall be accepted by the scaffolding erector so that scaffolds shall remain suitable for the purpose for which they are intended through out the works. The signed approval of the scaffolding shall be made visible at each ground level access point to the scaffolding. Working on unapproved scaffolding is strictly forbidden.

Care shall be taken that the load of any debris collecting on a scaffold does not exceed the loading for the design. The maximum permissible loading of the scaffolding shall be clearly visible at all ground level access points. All measures necessary shall be taken to prevent debris from being accidentally dislodged from the platform.

Steel scaffolding of trestle type, in accordance with local standards and regulations, including the supply of supports, maintenance, assembly, anchorage, dismantling etc. 15 cm toe boards shall be provided on all levels. Weatherproof sheeting or at least protective netting shall be provided on the outside of the scaffolding.

  - EXPLOSIVES:  Use of explosives is not allowed.

  - BURNING: Burning is not allowed.

  - CLEAN-UP:

    - Debris and Rubbish: The Contractor shall not utilise domestic garbage bins to dispose of construction debris or demolished material, but shall utilise appropriate containers for disposing of any and all construction related disposable materials and shall transport these off site as often as is necessary. Remove and transport debris and rubbish in a manner that will prevent spillage on streets or adjacent areas. Clean up any such spillage from streets and adjacent areas.

    - The site shall be cleaned: of all dust and debris following the completion of demolition works.

    - Regulations: Strictly comply with all relevant current site and Serbian hauling and disposal regulations.

# 2. EARTHWORK

EXCAVATION, BACKFILLING-FILLING, AND COMPACTING

# PART 1 - GENERAL

  - REFERENCES: The publications listed below form a part of this specification to the extent referenced.  The publications are referred to in the text by the basic designation only.

    - Applicable Serbian Laws and Norms

    - American Society for Testing and Materials (ASTM)Publications:

D 698-78	Moisture - Density Relations of Soil - Aggregate Mixtures Using  2.49 Kg hammer and 305 mm drop

D 1556-74	Density of Soil in Place by the Sand-Cone Method

D 1557-78	Moisture-Density Relations of Soils and Soil-Aggregate Mixtures, Using 10 4.54 kg hammer and 457mm Drop

D 2922-81	Density of Soil and Soil Aggregates in Place by Nuclear Methods (Shallow Depth)

  - DEFINITIONS:

1.2.1	Topsoil: Excavated material or undisturbed soil formations the fine grained, weathered materials on the surface or directly below any loose or partially decomposed organic matter. Topsoil may be a dark-coloured, fine, salty, or sandy material with a high content of well decomposed organic matter, often containing traces of the parent rock material. The material shall be representative of productive soil in the vicinity.

1.2.2	Hard Material: Weathered rock, dense consolidated deposits, or conglomerate materials which are not included in the definition of "rock" but which usually require the use of heavy excavation equipment, ripper teeth, or jack hammer for removal.

1.2.3	Unyielding Material: Rock or soil with cobbles in the trench bottom requiring covering of finer grain material or special bedding to avoid bridging in the pipe or conduit.

1.2.4	Unsatisfactory Material: Soil or other material identified as having insufficient strength or stability to carry intended loads on trench backfills without excessive consolidation or loss of stability. Also backfill material which contains refuse, frozen material, large rocks, debris, and other material which could cause the backfill not to compact.

1.2.5	Unstable Material: Material in the trench bottom which lacks the firmness to maintain alignment and prevent joints from separating in the pipe, conduit, or appurtenance structure during backfilling. This may be material otherwise identified as satisfactory which has been disturbed or saturated.

1.2.6	Borrow Materials: Shall be materials conforming to the requirements for fill. Contractor shall obtain such materials from sources outside the site, at his own expense.

    - Backfill: Material used in refilling a trench or other excavation which has been excavated on the site of this project and which is free from rocks, foreign materials and decomposed or clayey soils.

    - Lift: A layer or course of soil placed on top of unprepared sub grade or a previously prepared or placed soil in a fill or backfill.

    - Compaction: Any method of mechanically stabilizing a material by increasing its density at a controlled moisture condition. "Degree of Compaction" is expressed as a percentage of the maximum density obtained by required testing procedure.

  - SUBMITTALS:

1.3.1	Certified Laboratory Test Reports: Before delivery of materials, certified copies in triplicate of the reports of all tests required herein under materials and in referenced publications shall be submitted to the Supervisor or his representative for approval. Additional testing shall be submitted when the source of materials is changed. Certified test reports are required for the following:

(1)	Fill and backfill

1.3.2	Contractor Furnished Plans, Shop Drawings & Certifications: All trench excavations, 1.5 meters deep and over or where it is known that the in-situ soils lack the stability to hold trench vertical surfaces shall be provided with a shoring system. Bracing or shoring shall be carried -"As long as the excavation progresses”-. Contractor shall submit the following:

- Shoring Plan: List materials to be used on shoring system, indicate components that will remain after filling or backfilling.  Provide plans, sketches, or details along with calculations for all major excavation work.  Indicate sequence and method of installation and removal.

- Dewatering Plan: Describe methods for removing collected water from open trenches and diverting surface water or piped water away from work area.  Describe equipment and procedures for installing and operating the dewatering system indicated.  Describe the basic components of the dewatering system proposed for use, record performance and effectiveness of method or system in use and submit weekly.

  - SITE INFORMATION: It is expressly understood that the Client will not be responsible for any interpretation or conclusion drawn from provided soil reports. The data are made available is purely for the convenience of the Contractor. Additional soil borings and other exploratory operations may be made by the Contractor at no additional cost to the Client, provided such operations are approved by the Supervisor or his representative.

  - EXISTING UTILITIES: The location of the existing utilities shall be checked by the contractor if it is not indicated on the drawings. The Contractor shall physically verify the location and elevation of the existing utilities indicated prior to starting construction. The Contractor shall co-ordinate with the Supervisor or his representative for assistance in locating existing utilities. Approval for excavation shall be obtained from all relevant utility supply companies and a copy issued to the Supervisor or his representative.

  - MATERIAL STORAGE: Excavated materials classified as satisfactory soil material shall be stockpiled, where directed, until required for backfill or fill. Stockpiles shall be placed, graded, and shaped for proper drainage, and stored in a manner to prevent contamination and segregation. Materials required in the work shall be located and retained a sufficient distance from the edge of excavations to prevent such material falling or sliding back into the excavations and to prevent cave-ins. Waste materials, such as excavated materials classified as unsatisfactory soil material, trash, debris, and excess satisfactory soil material, shall be disposed of as advised by the Supervisor or his representative.

  - GENERAL REQUIREMENTS: execution shall be based on the following:

- That the surface elevations are as indicated on the drawings.

- That no pipes or other artificial obstructions, except those indicated, will be encountered.

- That hard material will not be encountered.

- The ground water indicated on the boring logs on the drawings are those existing at the time subsurface investigations were made and do not necessarily represent permanent ground water level.

In case the actual conditions differ substantially from those stated or shown, the provisions of the contract indicating an adjustment for changed conditions will not be applied, and all the charges will be the subject of Contractor. Hard material shall be defined as solid rock, firmly cemented non-stratified masses or conglomerate deposits possessing the characteristics of solid rock not ordinarily removed without systematic drilling and blasting and any boulder, masonry, or concrete except pavement, exceeding 0.4 cubic meters in volume.

# PART 2 - PRODUCTS

MATERIALS:

  - POROUS FILL: Capillary water barrier (indicated as porous fill) under concrete floor slabs shall consist of clean crushed stone, crushed gravel, or uncrushed gravel, 90-100 percent passing a 20 millimetres sieve and 0-5 percent passing a 4.75 millimetres sieve with sand equivalent of not less than 50. Granular fill may be composed of an appropriate combination of sand and crushed rock to meet the above gradation requirements. The capillary water barrier shall be placed directly on the sub grade. The barrier shall be constructed in layers not exceeding 10 centimetres in compacted thickness, and each layer shall be compacted with a minimum of two passes of a hand-operated plate type vibratory compactor.

  - SOIL MATERIALS:

2.2.1	Topsoil: Topsoil shall be free of subsoil refuse, stumps, and rocks over 3 centimetres, brush, weeds, and other materials detrimental to plant growth. Topsoil shall be obtained from an approved topsoil donor area

2.2.2	Satisfactory Soil Material: Satisfactory soil materials used as backfill for trenches, drains and for structures shall consist of native materials classified as well graded sands, silt sands or clayey sands which are free from debris, roots, wood, scrap materials and other vegetable matter and refuse.

2.2.3	Select Fill: Select Fill material shall be composed of sand or crushed rocks.  This material shall have the following gradation.

Sieve Designation                    Percentage passing (Weight)

40      	mm			90 to100

5      	mm (No. 4)		25 to 40

0.425 	mm (No. 40)		0-10

2.2.4	River Sand: The sand shall be natural river sand, made up of hard and clean particles, clear in colour and of spherical shape. The sand shall be clean and free from organic substances, clay, silt, and other impurities. The sand grade composition shall be from 0.5 mm to 1.0 mm.

2.2.5	Bedding Materials: Shall be fine and coarse sands with a graduation from 0 millimetres to 3.0 millimetres.

# PART 3 - EXECUTION

3.1	SURFACE PREPARATION:

3.1.1	Stockpiling Topsoil: Strip suitable soil from the site where excavation or grading is indicated and stockpile separately from other excavated material. Material unsuitable for use as topsoil shall be removed and disposed of off site. Locate topsoil so that the material can be used readily for the finished grading. Where excavated topsoil conforming to the material requirements is not sufficient Contractor shall provide borrow materials suitable for use as topsoil. Materials obtained from different sources shall be stockpiled separately.

3.2	EXCAVATION:

    - General: Excavation shall involve the removal and disposal of all materials encountered to obtain the specified sub grade elevations.

    - Removal of hard material: Notify the Supervisor or his representative immediately in writing if it becomes necessary to remove rock, or hard, unstable or otherwise unsatisfactory material to a depth greater than indicated. Immediately notify the Supervisor his representative if utility lines, not indicate on the as-built drawings are encountered during excavation

3.2.3	Protection of Persons and Property: All excavations shall be barricaded and posted with warning signs for the safety of persons. Warning lights shall be provided during hours of darkness. Structures, utilities, sidewalks, pavements and other facilities immediately adjacent to excavations shall be protected against damage including settlement, lateral movement, undermining, and washout. Barriers shall be placed so that vehicle loading adjacent to excavations is avoided. Stop logs shall be placed at the edge of excavations to prevent vehicles over running when reversing up to the edge of open excavations.

3.2.4	Excavation for Structures: Excavation for structures shall conform to the dimensions and elevations indicated within a tolerance of plus or minus 15 centimetres and shall extend a sufficient distance from footings and foundations to permit adequate working room of at least 50 cm for the placing and removal of concrete formwork, installation of services and other construction indicated, and for inspection. In excavations for footings and foundations, take care not to disturb the bottom of the excavation. Generally the final 10cm of excavations shall be removed shortly before placing of blinding concrete.

3.2.5	Trench Excavation: Excavation shall consist of excavating for foundations or utilities and plant distribution trenches. Make trench sides as nearly vertical as practicable except where sloping of sides is allowed

3.2.6	Excavation for Paved Areas: Excavation shall consist of excavating and grading for paved areas; excavating all unsuitable materials regardless of character, from the sub grade; and disposing of all excavated materials, as specified and in conformity with the lines, grades, cross sections, and dimensions shown on the drawings, and to replace unsatisfactory materials from other excavation and grading operations.

3.2.7	Unauthorized Excavation: Unauthorized excavation consisting of removal of materials beyond indicated sub grade elevations or side dimensions without the specific approval of the Supervisor or his representative shall be replaced at no additional cost to the Client. Unauthorized excavation under the footing or base of foundation shall be filled to the elevation of the footing or base of the foundation with lean concrete or well compacted select fill, without altering the approved top elevation. Elsewhere, unauthorized excavations shall be backfilled and compacted as per authorized excavations of the same classification

3.2.8	Stability of Excavations: Sides of excavations over 1.5m in depth shall be sloped to the angle of repose of the excavated material, or shall be shored and braced where sloping is not possible either because of space restrictions or stability of materials excavated. Sides and slopes of excavations shall be maintained until completion of backfill placement in a safe condition by scaling, benching, shelving, or bracing. Precautions shall be taken to prevent slides or cave-ins when sides of excavations are subjected to vibrations from vehicular traffic or the operation of machinery or any other source. Excavated material should not be closer to the edge of a supported trench than one-third of the trench depth. For unsupported trenches, this distance will depend on the depth of excavation, moisture content and cohesive strength of the material and profile of the excavation. Generally, excavated material should be placed outside a 45 degree slope line passing through the bottom of the excavation.

3.2.9	Shoring and Bracing: Materials used for shoring and bracing, such as sheet piling, uprights, and stringers and cross braces, shall be in good serviceable condition. All timber used shall be sound and free from large or loose knots. Shoring and bracing in excavations shall be maintained regardless of the length of time excavations remain open. All shoring and bracing shall be carried down with the excavation. Wherever subsequent removal of sheet piling could permit the lateral movement of soil under adjacent structures, sheet steel piling or pressure creosoted timber sheet piling shall be used, and left permanently in-place, and cut off as required.

    - Water Removal: Excavations shall be performed in a manner to prevent surface water and subsurface or ground water from flowing into the excavations and to prevent water from flooding the project site and surrounding area. Water shall not be permitted to accumulate in excavations.

All water shall be removed from excavations by approved dewatering methods so that softening of foundation bottom, undercutting of footings, and soil changes detrimental to the stability of sub grades and foundation will not occur. Pumps, sumps, suction and discharge lines and other dewatering system components shall be provided and maintained as necessary to convey the water away from the excavations. Dewatering operations shall be continuous until the completion of backfill placement and until construction subject to water pressure has obtained the full specified strength. In all instances, dewatering operations shall continue as long as water can enter or accumulate in the excavations. Water removed from excavations and rain water shall be conveyed to collecting channels, as approved by the Supervisor or his representative. Temporary drainage ditches and other diversions shall be provided and maintained outside the excavation limits for each structure. The use of trench excavations for site utilities as temporary drainage ditches will not be permitted.

3.3	BACKFILL & FILL:

    - General: Backfill shall consist of the placement of specified backfill material, in layers, in the excavations to the indicated sub grade elevations, for each area classification listed below. Fill shall consist of placement of specified fill materials in layers, over the ground surface to the indicated elevations, for each area classified below:

    - Backfill and Fill Materials: Soil materials for backfill and fill shall be free of clay clods, rock or gravel larger than 6 centimetres. Debris waste, frozen materials and other deleterious matter of any dimension shall be removed. Filling shall be made with satisfactory soil materials as follows:

| ### Area Classification | Backfill or Fill Material |
| --- | --- |
| In all excavations, unless otherwise specified hereinafter | Excavated or borrow material that has been sampled, tested and approved as "Satisfactory Soil Material.” |
| Under pedestrian paths | Sub-base material, or excavated or borrow material that has been sampled, tested and approve as "Satisfactory Soil Material". |
| Under grassed areas | Sub-base material, or excavated or borrow material that has been sampled, tested and approve as "Satisfactory Soil Material'. |

3.3.3	Preparation Prior to Backfill Placement: Excavations shall be backfilled as promptly as the work permits, but not until completion of the following:

- Approval of construction below finish grade.

- Inspection, testing, approval, and recording location of underground utilities.

- Removal of concrete formwork.

- Removal of shoring and bracing, and backfilling of voids with satisfactory soil material temporary sheet piling driven below the bottom of structures and utilities shall be cut off and removed in a manner to prevent settlement of the structure or utilities.

- Removal of rubbish and debris.

3.3.4	Preparation of Ground Surface to Receive Fill: Vegetation, debris, unsatisfactory soil materials, obstructions, and deleterious materials shall be removed from ground surface prior to the placement of fills. Sloped surfaces steeper than one vertical to four horizontal shall be ploughed, stripped, or broken-up in such manner that fill material will bond with the existing material. When the ground surface has a density less than that specified for the particular area classification, the ground surface shall be broken-up, pulverized, and moisture-conditioned to near optimum moisture content of the soil material and compacted to the required depth and percentage of maximum density

3.3.5	Backfilling Trenches: Trenches shall be carefully backfilled with materials specified, and deposited in 15 centimetres maximum layers, loose depth. Borrow materials shall be obtained from approved sources outside the site. The source of borrow material shall be the Contractor’s responsibility. Utility surround material (as per bedding) shall be brought up evenly on both sides of the utility line for its full length, and shall be thoroughly and carefully tamped until utility line has a cover of not less than 30 centimetres. Care should be taken not to damage pipe or special coatings on utility line. Warning utility identification bands appropriate to the utility line shall be laid in the trench upon this layer. The remainder of backfill material shall be then deposited in the trench in 30 centimetres maximum loose layers and compacted by hand operated mechanical tampers. Trenches and excavation pit improperly backfilled or where settlement occurs, shall be reopened to depth required to obtain the specified compaction, then refilled and compacted with the surface restored to the required grade and compaction. To prevent material loss through drainage effects in the bedding and surround material in trenches of steep gradient, barriers of impermeable material such, as puddle clay, shall be installed across the entire width of the excavation at least 30m centres measured along the line of the trench.

3.3.6	Preparation of Planted Areas: Planted areas shall be graded to the dimensions, elevations, and cross sections indicate on the Drawings. The top 10 centimetres, or as otherwise shown, of such areas shall consist of top soil as defined herein, which shall be lightly compacted. All material below the topsoil layer shall be compacted as specified for sub grade preparation. The topsoil shall be uniformly distributed on the designated areas and evenly spread to an average thickness of 10 centimetres with a minimum thickness of 8 centimetres. Prior to placing the topsoil, the sub grade, wherever exceedingly compacted by traffic or other causes, shall be loosened by disking or by scarifying to a depth of at least 6 centimetres to permit bonding to the sub grade. The spreading shall be performed in such a manner that planting can proceed with little additional soil preparation or tillage. Any irregularities in the surface resulting from top soiling and other operations shall be corrected in order to prevent the formation of depressions where water will stand. Topsoil shall not be placed when the sub grade is excessively wet, extremely dry, or in a condition otherwise detrimental to proper grading.

3.4	COMPACTION:

3.4.1	General: The compaction of soil materials for backfills and fills shall be performed by use of the specified compaction equipment that is suitable for the soil material being compacted and for use in the location of the work area. Soil compaction shall be controlled during construction for compliance with the percentage of maximum density for each area classification as specified.

3.4.2	Compaction Equipment: All compaction equipment shall be of suitable size and number, and in satisfactory working condition to complete the work on schedule. Compaction equipment shall consist of sheep foot rollers, pneumatic type rollers, tamper rollers, vibrating tampers, or other compaction equipment suitable for the soil material being compacted and capable of obtaining the required density throughout the entire layer being compacted.

3.4.3	Placement and Compaction: Backfill and fill materials shall be placed in layers not more than 15 centimetres in loose depth. Before compaction, each layer of backfill or fill material shall be moistened or aerated as necessary to provide the optimum moisture content of the soil material. The backfill and fill materials shall be compacted to 95% of the maximum density to a depth of no less than 25 centimetres for each area classification as specified. No backfill or fill material shall be placed on surfaces that are muddy, frozen, or contain frost, or ice. Backfill and fill materials adjacent to structures shall be brought-up evenly around the structure equal to the depth of backfill or fill material (measured from the bottom of footing or foundation or retaining wall) to final grade, shall be compacted with power-driven hand tampers. Porous fill shall be compacted with a minimum of two passes with a handheld vibratory compactor.

3.4.4	Degree of Compaction: Except where otherwise specified, the degree of compaction and density shall be determined and controlled in accordance with requirements of ASTM D 698.

3.4.5	Field determination of the density of soils in place shall be in accordance with ASTM D 2922.

3.4.6	Compaction: The sub grade of soils in cut shall have a density of at least 95 % of the maximum density to a depth of 25 centimetres below the sub grade surface. Fill, embankment, and/or backfill under concrete floor slabs and the upper 30 centimetres under paved areas shall be compacted to not less than 95 % of the maximum density; other backfill adjacent to and not supporting and structural elements to at least 90 %. The upper 30 centimetres of trenches shall be compacted to at least 95 % maximum density.

3.4.7	Moisture Control: Provide equipment capable of adding measured amounts of moisture to the top soil material as determined by moisture-density relation tests. The moisture content in the soil material at the time of compaction shall be within plus or minus two percent of optimum. For unsatisfactory soil materials where the sub grade or layer of soil material must be moisture conditioned before compaction, the required amount of water shall be uniformly applied to the surface of sub grade or layer of soil material in such manner as to prevent free water appearing on the surface during or subsequent to compaction operations. All soil material that is too wet to permit compaction specified percentage of maximum density shall be removed and replaced with satisfactory soil material. Soil material that has been removed because it is too wet to permit compaction may be stockpiled or spread on the surface where advised by the Supervisor or his representative and permitted to dry, assisted by digging, harrowing or pulverizing, until the moisture content is reduced to a satisfactory value as determined by moisture-density relation tests, after which the soil material may be used in compacted backfill or fill.

3.5	GRADING:

3.5.1	General: All areas within the limits of grading under this section, including adjacent transition areas, shall be uniformly graded. The finished surface shall be smooth within the specified tolerances, compacted, and within the tolerance specified below for each area classification, compacted as specified, and free from irregular surface changes.

3.5.1.1	Grassed Areas: The finished surface of areas to receive topsoil shall be not more than 3 centimetres above or below the indicate sub grade elevations.

      - Pedestrian paths Areas: The surface of areas under walks shall be shaped to line, grade and cross-section, and finished surface shall not be above nor more than 1-2 centimetres below the indicate sub grade elevation

3.6	REMOVAL FROM SITE: Waste materials, including excavated material classified as unsatisfactory or excess soil material, trash and debris shall be removed from site and legally disposed of at expense of the Contractor at the site indicated by the beneficiary.

3.7	PROTECTION OF GRADED AREAS: Newly graded areas shall be protected from traffic and erosion and shall be maintained free of trash or debris. Repair and re-establish grades in settled, eroded, and rutted areas to the satisfaction of the, Supervisor or his representative.

3.8	RECONDITIONING COMPACTED AREAS: Where approved compacted areas are disturbed by subsequent construction operation or adverse weather, the surface shall be scarified, re-shaped, and compacted as specified hereinbefore to the required density prior to further constructions thereon, re-compaction over underground utilities shall be by hand tamping.

3.9	FIELD SAMPLING AND TESTING:

3.9.1	Testing: All testing shall be conducted by the Contractor as specified herein at the expense of the Contractor.

3.9.2	Granular Fill Testing: Gradation test shall be made on each sample. Tests shall be performed for each 100 cubic meters of material used or fraction thereof and whenever the source is changed.

Compaction Testing: Compaction tests shall be made in locations as selected by the Supervisor or his representative as follows:

| ##### Materials | Test Frequency |
| --- | --- |
| (1) Fill and Backfill | 1 per lift per every 100 m2 |
| (2) Sub grade (existing in-place density) | 1 per lift per every 800 m2 |
| (3) Fill in trenches under pavement | 1 per lift per every 200 m of trench |

# 3. CAST-IN-PLACE CONCRETE STRUCTURES

# PART 1 - GENERAL

1.1	REFERENCES: The publications listed below form a part of this specification to the extent referenced. The publications are referred to in the text by the basic designation only.

    - Applicable Serbian Laws and Norms

1.1.2	European Norms and Standards:

| DIN EN 12350-1 : 2001 | On-site concrete sampling |
| --- | --- |
| DIN EN 12390-2 | Concrete Test Specimens Preparation and Curing |
| DIN EN 12504-1 | Sampling of Hardened Concrete |
| DIN EN 12390-3 | Destructive Testing of Concretes |
|  | Compression Test |
| DIN EN 934-2 | Concrete Admixtures - Definition and Classification |
|  | Concrete Admixtures - Air Entrainments Suitable Applications and Control Methods |
| DIN EN 206 | Concrete |
| DIN EN 12350-2 | Fresh Concrete - Determination of Consistency - Slump Test |
| DIN 18331 | Construction contract procedures (VOB) - Part C: General technical specifications in construction contracts (ATV); Concrete work |
| DIN 18541-1 | Thermoplastics sealing strips for sealing joints in in-situ concrete - Concepts, geometry and dimensions |
| DIN 18541-2 | Thermoplastics sealing strips for sealing joints in in-situ concrete - Requirements, testing and inspection |

1.2	SUBMITTALS: Submit the following:

1.2.1	Manufacturers Data: Submit for approval:

- Materials for curing concrete

1.2.2	Shop drawings: Submit shop drawings for the following. Reproductions of contract drawings as shop drawings are unacceptable.

- Shop Drawings for Reinforcing Steel: Provide bending diagrams, assembly diagrams, splicing and laps of bars, shapes, dimensions, and details of bar reinforcing, accessories, and concrete cover. Do not scale dimensions from structural drawings to determine lengths of reinforcing rods.

- Shop Drawings for Formwork: Furnish schedule of formwork.

1.3	DELIVERY AND STORAGE OF MATERIALS: Packaged materials shall be delivered to the project site in their original, unopened package or container. Packages will bear manufacturer's label clearly identifying manufacturer's name, brand name, material, weight or volume, and other pertinent information and stored in their original, unbroken package or container in a weather tight and dry place, until ready for use. Unpackaged aggregates shall be stored in a manner as to avoid excessive segregation or contamination with other materials or other sizes of aggregates. Store reinforcement of different sizes and shapes in separate piles, or racks raised above the ground to avoid excessive rusting.  Protect from contaminants such as grease, oil, and dirt. Provide for accurate identification after bundles are broken and tags removed. Ready–mix concrete shall not be delivered until forms, reinforcement and other embedded items are in place and ready for concrete to be placed. Prefabricated hollow slabs will be delivered and stored on site without being damaged.

  - CONCRETE QUALITIES: Concrete shall be in strict accordance with applicable portions of DIN EN 206-1. The concrete shall have a 28-day compressive strength

# PART 2 - PRODUCTS

2.1	CONCRETE:

2.1.1	Strength Requirements: Concrete classified as follows shall be proportioned and mixed for the strength characteristics in accordance with DIN EN 206-1.

Concrete should be fast dry, polymer modified, calcium cement-based, high early strength, self mortar and underlayment. , that can be poured or pumped from 3 mm-30mm. High early strength cement may be used only with specific written approval by the Supervisor . The quantity of cement per cubic meter of concrete is defined in DIN EN 206-1

2.1.2	Concrete Qualities:

2.1.2.1	General: Concrete shall have a minimum compressive strength as indicated, in the table of paragraph 2.1.1 unless specified otherwise.

2.1.2.2	Concrete Consistency: Tests of the concrete consistency, esp. the slump test shall be carried out in accordance to DIN EN 12350-2

2.1.3	Ready-Mixed Concrete: DIN EN 206-1. Ready-mixed concrete is defined in this specification as concrete produced regularly by a commercial establishment and delivered to the purchaser in the plastic state. Ready-mixed concrete may be used provided that:

- The plant has sufficient capacity and transportation equipment to deliver the concrete at the rate desired.

- The interval between batches for a pour shall not exceed 30 minutes

- The time elapsing between the introduction of the mixing water to the cement and aggregates, or the cement to the aggregate, and placing of the concrete in final position in the forma shall not exceed one hour.

Ready mixed concrete shall be mixed and delivered by means of one of the following methods:

2.1.3.1	Central Mixing: Central mixing by mixing the concrete completely in a stationary mixer at the plant and transporting the mixed concrete to the site of the work in a truck agitator or a truck mixer operating at agitating speed.

      - Transit Mixing: Shall be mixing completely in a truck mixer. Mixers and agitators shall be operated within the limits of capacity and speed of rotation designated by the manufacturer, and shall be capable of reproducing a homogeneous mixture of uniform colour. Weighing and moisture determination shall be as specified herein.

2.2	CEMENT: The cement type shall be selected in accordance to DIN EN 197-1

2.3	WATER: Water shall be fresh, clean, and potable.

2.4	AGGREGATES: The aggregate shall be selected in accordance to DIN EN 206-1. Generally, all natural sands and gravel, crushed stones or other products previously proven satisfactory by practice, can be used as aggregates in mixing concrete provided the requirements specified are met. Aggregates shall not contain any substance which may be reactive with the alkalinity of the cement.

2.5	ADMIXTURES: Additives for concrete shall conform to DIN EN 206-1. Super plasticizer admixture shall come from the same manufacturer as other admixtures, or shall be certified to be compatible with all admixtures.

2.6	REINFORCEMENT:

2.5.1	Reinforcing Bars: according to ENV 10080 shall be a ribbed reinforcing steel with a characteristic yield strength of 500 N/mm² and a minimum diameter of 8mm.

2.5.2	Welded Wire Fabric (mesh): shall be a ribbed steel with a characteristic yield strength of 500 N/mm². It shall be electrically welded steel wire fabric for concrete reinforcement. Unless otherwise indicated on the drawings, shall be l0 x l0 centimetres, 8 millimetres in diameter, for general construction.

2.5.3	Tie Wires: Shall be 1.2 millimetres in diameter, black annealed wire.

2.7	CONCRETE PROTECTIVE COVER OVER REINFORCEMENT: The minimum concrete reinforcement cover shall be as follows:

- Sides and bottom of footings

Cast against the earth	50 millimetres

- Concrete exposed to the weather	35 millimetres

- Concrete inside rooms	30 millimetres

2.8	END SPLICES FOR REINFORCING BARS: End laps of reinforcing bars shall be not less than 40 bar diameters, unless otherwise indicated.

  - WELDED-WIRE MESHES LAP SPLICES: Minimum one full mesh plus 5 centimetres overhang on each end, unless otherwise indicated.

# PART 3 - EXECUTION

3.1	FORMS:

3.1.1.	General: Set forms true to line and grades and make mortar-tight.  Chamfer above grade exposed joints, edges, and external corners of concrete 20 millimetres unless otherwise indicated. Before concrete placement, coat the contact surfaces of forms with a no staining form coating compound. Do not use mineral oil on formed surfaces to be painted. Prevent concrete damage during form removal. Concrete for footings may be placed in excavations without forms upon inspection and approval by the Supervisor or his representative. Excavation width shall be a minimum of 10 centimetres greater than indicated.

3.2	PLACING REINFORCEMENT AND MISCELLANEOUS MATERIALS:

3.2.1	General: Provide bars, welded wire fabric, wire ties, supports, and other devices necessary to install and secure the reinforcement. Clean all reinforcement of loose, flaky rust, scale, grease, mortar, grout or other coatings which may destroy or reduce its bond with the concrete.  Reinforcement which has bends not shown on the project drawings or on approved shop drawings or which is reduced in section by rusting shall not be used. Placement of all reinforcement shall be inspected and approved by the Supervisor or his representative prior casting the concrete.

Placing: Reinforcement shall be accurately located in the forms, and firmly held in place, before and during the placing of concrete. This means of bar supports adequate to prevent displacement during the course of construction and to keep the steel at a proper distance from the forms.

Reinforcing bars shall be supported by concrete or metal chairs, by metal spacers, by metal bangers, or other approved supports. End laps for reinforcing bars shall not be less than 40 bar diameters unless indicated otherwise.

For slabs on grade (over earth) and for footing reinforcement, bars or mesh shall be supported on precast concrete blocks, spaced at intervals required by size of reinforcement used, to keep reinforcement the minimum height specified above the underside of slab or footing. Welded wire fabric shall be supported as required for reinforcing bars. Reinforcement shall be secured to supports by means of tie wire and shall be accurately placed and securely tied at all intersections with 1.6-millimeter diameter annealed wire.  Wire-tie ends shall point away from the form. Welding of crossing bars shall not be permitted for assembly of reinforcement unless specifically authorized by the Supervisor or his representative.

3.2.3.	Setting Miscellaneous Material: Place and secure anchors and bolts, pipe sleeves, conduits, and other such items in position before concrete placement. Plumb anchor bolts and check location and elevation. Temporarily fill voids in sleeves with readily removable material to prevent the entry of concrete.

  - MATERIALS

Steel reinforcement shall be high tensile, high-bond deformed bars complying with the requirements of Serbian Standards or EU norms and with a guaranteed yield stress of not less than 420 N/mm2.

Mild plain steel bars should have a guaranteed yield stress of 250 N/mm2.

Mesh reinforcement shall be in accordance to Serbian Standards or EU norms

  - MINIMUM TEST PROCUREMENT

Every 50 tons samples shall be taken and tested for:

- 	Tensile Strength

- 	Yield Point

- 	Elongation after fracture

-	Behaviour in the bend test

- 	Behaviour in the rebound test

- 	Deviation from cross section

- 	Bond test

-	Chemical composition

  - MEASURING, MIXING, TRANSPORTING, AND PLACING CONCRETE:

Concrete shall be poured monolithically.

    - Ready-mix Concrete: Ready mix concrete furnished in revolving concrete mixers on trucks will be acceptable provided the requirements of DIN EN 206-1, and the following are met:

The ready-mixed concrete manufacturer shall furnish duplicate delivery tickets with each load of concrete delivered to the project site, one for the Supervisor or his representative and one for the Contractor.  In addition to the requirements of DIN EN 206-1, the delivery tickets shall provide the following information:

- Type and brand of cement

- Cement content per cubic meter of concrete

- Maximum size of aggregate

- Total water content expressed by Water/Cement Ratio

- Truck identification

- Volume of concrete in truck

- Batch time

Machine mix concrete and provide mandatory batch ticket information for each load of ready mix concrete.  Begin mixing within 30 minutes after the cement has been added to the aggregates.

3.5.2	Placing: Place concrete within 90 minutes of either addition of mixing water to cement and aggregates or addition of cement to aggregates if the air temperature is less than 30 degrees Centigrade.  Reduce mixing time to 60 minutes if the air temperature is greater than 30 degrees Centigrade. Additional water may be added, provided that both the specified maximum slump and water-cement ratio are not exceeded. Do not place concrete when: (a) weather conditions prevent proper placement and consolidation; (b) in uncovered areas during periods of precipitation; and (c) in standing water. Prior to placing concrete, remove dirt, construction debris, water, snow, and ice from within the forms. Concrete shall not be permitted to free fall over a height of more than 3 m to prevent segregation.

Consolidate concrete slabs greater than 10 centimetres in depth with high frequency, internal, mechanical vibrating equipment supplemented by hand spading and tamping.

Consolidate concrete slabs 10 centimetres or less in depth by hand rodding, tamping and spading.

3.5.2.1	Concrete Pumps: Concrete may be conveyed using concrete pumps only after written approval by the Supervisor or his representative. In requesting approval, the Contractor shall submit his entire plan of operation from time of discharge of concrete from the mixer to final placement in the forms. Concrete pumps shall be operated and maintained so that a continuous stream of concrete is delivered into the forms without air pockets, segregation, or change in slump exceeding 5 centimetres.

3.3.2.2	Concrete Vibration: Immediately after placing, each layer of concrete shall be vibrated using internal concrete vibrators supplemented by hand spading and tamping. Tamping or other external vibrations of forms are not permitted. Vibrators shall not be used to transport concrete inside forms.

Internal vibrators submerged in concrete shall maintain a minimum frequency of not less than 8000 vibrations per minute. Duration of vibration shall be limited to time necessary to produce satisfactory consolidation, generally from 5 to 15 seconds. Vibrators shall be applied at uniformly spaced points of 45 centimetres.

3.3.3	Cold Weather: Provide and maintain 10 degrees Centigrade minimum concrete temperature. Do not place concrete when the ambient temperature is below 5 degrees Centigrade. Cover concrete and provide with a source of heat sufficient to maintain l0 degrees Centigrade minimum while curing.

3.3.4.	Hot Weather: Provide and maintain 32 degrees Centigrade maximum concrete temperature. Cool ingredients before mixing, or use other suitable means to control concrete temperature to prevent rapid drying of newly placed concrete. Shade the fresh concrete and start curing as soon as the surface of the fresh concrete is sufficiently hard to permit curing without damage.

    - Construction Joints: if joints are necessary, their position will be agreed with the Supervisor

3.4	SURFACE FINISHING:

3.4.1	Defects: Repair formed surfaces by removing minor honeycombs, pits greater than 2 square centimetres surface area or 6 millimetres maximum depth, or otherwise detective areas. Provide edges perpendicular to the surface and patch with non-shrink grout. Patch ties holes and defects when the forms are removed. Concrete with extensive honeycomb (including exposed steel reinforcement, cold joints, entrapped debris, separated aggregate or other defects) which affect the serviceability or structural strength will be rejected, unless correction measure is submitted and approved by the Supervisor or his representative. The surface of the concrete shall not vary more than the allowable tolerances specified herein. Exposed surfaces shall be uniform in appearance and, finished to a smooth form finish unless otherwise specified.

3.4.2	Floated Finish: Place, consolidate, and immediately strike off concrete to obtain proper contour, grade, and elevation before bleed water appears. Permit concrete to attain a set sufficient for floating and supporting the weight of the finisher and equipment. Do not spread dry cement over slab surface to absorb bleed water. Do not introduce a "topping mix" over the floated finish of a slab to achieve a smooth finish.

3.5	CURING AND PROTECTION: Protect concrete from injurious action by sun, rain, flowing water, frost, mechanical injury, tire marks, and oil stains. Do not allow concrete to dry out from time of placement until the expiration of the curing period. Forms may be removed 48 hours after concrete placement.  Prohibit foot and vehicular traffic and other sources of abrasion for not less than 72 hours after finishing operations.

    - Moist Curing and Water Spraying: Provide uniform and continuous application of water spray throughout the curing period. Curing shall be by direct application of water, accomplished by fog spraying or sprinkling. Flood surface with water after concrete has sufficiently hardened so, as not to be damaged by the spray. Apply water spray a minimum of two to three times a day and as often as ambient temperature and weather conditions dictate, for the minimum curing period of seven days. Allow concrete to cure under this method for a minimum period of seven days.

    - Levelling of the supporting surface: the surface must be levelled removing all irregularities.

    - Insitu Slab: to guarantee the continuity of adjacent elements a slab must  be cast in concrete of resistance strength class C 25/30 (cube strength 30N/mm²), with reinforced welded wire mesh 20x20cm diameter 8  mm, over the panels.

3.6	FIELD QUALITY CONTROL SAMPLING AND TESTING:

Collect samples of fresh concrete to perform tests specified.

3.6.1	Slump Tests: DIN EN 12350-2. Take samples during concrete placement.  Perform slump tests at (a) commencement of concrete placement, (b) when test cylinders are made, and (c) for each batch or every 10 cubic meters of concrete.

    - Compressive Strength Tests: DIN EN 12390-3. Make minimum of five test cubes with a measurement of 150/150/150mm for compressive test. Leave them in a water basin. Test the cubes at 28 days and hold one cube in reserve. Provide concrete cubes for compressive tests not less than once for each 30 cubic meters of concrete. Testing of specimens for compressive strength shall be in accordance with DIN EN 12390-3.  Not more than 10% of the specimens tested shall have a compressive strength less than specified. When the results of the test of the control specimens indicate that the concrete as placed does not meet specification requirements or where there is evidence that quality is below specification requirements, core boring shall be accomplished in conformance with DIN EN 12504-1. The cost of such tests will be borne by the Contractor. Strength of cores from each member or area shall be considered satisfactory if their average is equal to or greater than 90 percent of the design compressive strength of the class of concrete. Remove all concrete not meeting this strength criteria and replace with new acceptable concrete at no additional cost to the Client. Repair core holes with non-shrink grout. Match colour and finish with adjacent concrete surface.

    - Surface Test: After curing, test the surface of the pavement with a straightedge (minimum length 3 metres) which will reveal any irregularities in the concrete surface. Remove and replace the concrete, mechanically grind the concrete surface, or correct the surface as approved, of any portion of the pavement which shows irregularities greater than 3 millimetres in 3 metres, or irregularities in a transverse direction greater than 6 millimetres in 3 metres.

    - Test Results: Submit test results as part of the "Daily Report to Inspector" except that Compressive Test results shall be reported by separate correspondence or submittal.

    - STRIPPING OF FORMWORK: Formwork shall not be removed before the concrete has attained sufficient strength to support its own mass and any loads that may be imposed on it. This condition shall be assumed to require formwork to remain in place, after placing of the concrete, or the appropriate minimum period of time given in the Table below, unless the contractor can prove to the satisfaction of the Supervisor or his representative that shorter periods are sufficient to fulfil this condition.

Minimum periods before striking of formwork

Type of formwork	Surface temperature of concrete

16°C	7°C

Vertical formwork to columns,	3 days	5 days

Walls and large beams	2 days	3 days

(Lateral formwork)

False work to slabs	4 days	7 days

Props to slab false work	11 days	14 days

False work to beams	8 days	14 days

Props to beam false work	15 days	21 days

Note:

The Supervisor or his representative may allow a shorter period, when using Rapid Hardening Cement.

For cold weather periods should be increased by ½ day for each day the temperature falls between 7°C and 2°C, and one day for each day on which the temperature drops below 2°C.  Formwork shall be removed carefully so that damage to the concrete is avoided.

# 4.  STRUCTURAL METAL WORKS

# PART 1 - GENERAL

1.1	REFERENCES: The publications listed below form a part of this specification to the extent referenced.  The publications are referred to in the text by the basic designation only.

    - Applicable Serbian Laws and Norms:

1.1.2	European Norms and Standards:

DIN V ENV 1993	Design of Steel Structures

DIN EN 10025	Hot rolled products of structural steels

1.2	SUBMITTALS: Submit the following:

1.2.1	Shop drawings: Before to raise the metal structures submit shop drawings to Supervisor or his representative for approval. Reproductions of contract drawings as shop drawings are unacceptable.

Provide type, grade, dimensions, and details of truss beams and built-up girders, including reinforcing, accessories, anchorages, bending diagrams, assembly diagrams, splicing and laps of bars, shapes. Do not scale dimensions from structural drawings to determine lengths of reinforcing rods.

1.2.2	Certificates of Compliance: Submit the following manufacturer’s certifications:

- Steelwork

- Bolts and nuts

- Surface painting material

- Electrodes

- Hot deep galvanised

1.3	QUALITY CONTROL

- Welding inspectors shall be certified by all relevant Serbian Standards or equivalent EU norms as per the approval of the Supervisor or his representative.

- The Contractor shall carry out at his own cost inspection and testing of splices introduced by fabricator in steel sections at random locations

- Non-destructive Testing of Welded Connections:

The Contractor shall carry out at his own cost non-destructive testing of welded connections chosen at random as follows:

- 1.10% of moment connections involving use of fillet weld, by Magnetic Practice Inspection.

- All moment connections and all connections in direct tension-involving use of Full Penetration Groove welds, by ultrasonic testing.

- Where moments are transferred by either fillet welds or groove welds into end plates in "T", “U” and “L” joint configurations, examine base metal for lamellar tearing or cracking, by ultrasonic testing.

- For installing tolerances has to be applied EU and Serbian standards.

# PART 2 - PRODUCTS

  - STEEL MATERIALS

- New materials shall be provided in accordance with reference standards, of strength and quality noted on Structural Drawings.

- Cold formed channels:

To be fabricated to Serbian Standards or EU norms.

- Anchor Bolts: to Serbian Standards or EU norms.

- Welded studs for embedded plates shall be in accordance with Serbian Standards or EU norms.

- The Contractor must ensure compatibility with the spray fireproofing prior to applying the primer.

High strength bolts to be used as per Serbian and EU codes and also as noted on Drawings.

  - PAINTING

- All structural steel has to be hot deep galvanised as it is always shown on the drawings

  - FIRE PROOFING

All structural steel shall be fire proofed to achieve 1hr fire rating. The method of fire-proofing shall be compatible with the final finish and be to the approval of the Supervisor or his representative.

  - STEEL MATERIALS

2.4.1	Structural Steel

The structural steel has to be chosen according to EN 10025:

Steel grade		Yield strength                    Tensile strength

Fe 360 		    	235 N/mm² 		360 N/mm²

Fe 430			275 N/mm²		430 N/mm²

Fe 510 		   	355 N/mm²		510 N/mm²

  - BOLTS, NUTS, AND WASHERS

2.5.1	Bolts: EN 24014, EN 24016, EN 24017 and EN 24018

2.5.2	Nuts: EN 24032, EN 24034, ISO 7413

2.5.3	Washers: ISO 7089, ISO 7090, ISO 7091

# PART 3 - EXECUTION

  - INSTALLATION: After final positioning of steel members, provide full bearing under base plates and bearing plates using non shrink grout. Place non shrink grout in accordance with the manufacturer's instructions. Put under the structural steel as shown on the drawing a type of shim approved, which is dimensioned on drawings.

  - CONNECTIONS: Do not tighten anchor bolts set in concrete with impact torque wrenches. Punch, sub punch and ream, or drill bolt holes. Bolts, nuts, and washers shall be clean of dirt and rust and lubricated immediately prior to installation.

3.2.1 Bolts: Bolts shall be tightened to a "snug tight" fit. "Snug tight" is the tightness that exists when plies in a joint are in firm contact. If firm contact of joint plies cannot be obtained with a few impacts of an impact wrench, or the full effort of a man using a spud wrench, contact the Supervisor or his representative for further instructions.

Test a minimum of 3 bolt, nut, and washer assemblies

  - WELDING:

Use only shielded metal arc welding and low hydrogen electrodes steel. Provide qualified welders, welding operators. Removal of Temporary Welds, Run-Off Plates, and Backing Strips Test locations shall be selected by the Supervisor or his representative. If more than 20 percent of welds made by a welder contain defects identified by testing, then all welds made by that welder shall be tested by radiographic or ultrasonic testing, as approved by the Supervisor or his representative.

# 5.  MASONRY

# PART 1 - GENERAL

  - REFERENCES:

The publications listed below form a part of this specification to the extent referenced. The publications are referred to in the text by the basic designation only.

1.1.1	Applicable Serbian Laws and Norms:

1.1.2	European Norms and Standards:

Brick/ Block work

DIN EN 771-1	Specification for masonry units - Part 1: Clay masonry units

DIN EN 771-2	Specification for masonry units - Part 2: Calcium silicate masonry units

DIN EN 771-3	Specification for masonry units - Part 3: Aggregate concrete masonry units (Dense and light-weight aggregates)

DIN EN 771-5	Specification for masonry units - Part 5: Manufactured stone masonry units

DIN EN 771-6	Specification for masonry units - Part 6: Natural stone masonry

DIN EN 998-1	Specification for mortar for masonry - Part 1: Rendering and plastering mortar

DIN EN 998-2	Specification for mortar for masonry - Part 2: Masonry mortar

DIN EN 1015-1	Methods of test for mortar for masonry - Part 1: Determination of particle size distribution (by sieve analysis)

DIN EN 1015-11	Methods of test for mortar for masonry - Part 11: Determination of

flexural and compressive strength of hardened mortar Ancillaries

DIN EN 845-1 	Specification for ancillary components for masonry - Part 1: Ties, tension straps, hangers and brackets

DIN EN 845-2 	Specification for ancillary components for masonry - Part 2: Lintels

DIN EN 845-3 	Specification for ancillary components for masonry - Part 3: Bed joint reinforcement of steel meshwork

  - SUBMITTALS:

| Submit the following | Shop Drawings |
| --- | --- |
|  | Reinforcing steel |
|  | Movement joints |
|  | Masonry accessories |
|  | Reinforcement |
| Submit for each type samples: | Masonry units; |
|  | Stone samples |
|  | Concrete masonry units |
|  | Building brick |
|  | Facing brick |
|  | Mortar strength and properties |
|  | Masonry cement |
|  | Grout |
|  | Test Reports |
|  | Certificates |
|  | Manufacturer's Instructions |

  - QUALITY ASSURANCE

1.3.1 	Appearance: Do not change source or supply of materials after brick manufacturing work has started. Bricks shall be manufactured at one time and from the same batch. Blend all brick to produce a uniform appearance when installed. An observable "banding" or "layering" of colours or textures caused by improperly mixed brick is unacceptable.

1.3.2	Testing: Masonry strength shall be determined in accordance with Supervisor or his representative and the cost of testing shall be paid by the Contractor.

1.3.3	Field Samples: Masonry Panel Requirements At the project site submit for approval by the Supervisor or his representative, a sample masonry panel approximately 1200 mm long by 1200 mm high showing the workmanship, coursing, bond, weep holes, flashing, masonry ties and anchors, rigid-board insulation and tooling of joints. The sample panel shall be protected from damage and shall remain at the site until masonry work is complete and approved, at which time the panel shall be removed from the site. Masonry work shall match the approved sample.

  - DELIVERY, STORAGE, AND HANDLING:

Deliver cementations materials to the site in unbroken containers, plainly marked and labelled with manufacturers' names and brands. Store cementations materials in dry, weather-tight sheds or enclosures and handle so as to prevent entry of foreign materials and damage by water or dampness. Store masonry units off the ground and handle with care to avoid chipping and breakage. Protect materials from damage and, except for sand, keep dry until used. Cover sand to prevent intrusion of water and foreign materials and to prevent drying. Do not use materials containing frost or ice. Protect moisture controlled units from rain and ground water.

  - SCHEDULING:

Coordinate masonry work with the work of other trades to accommodate built-in items and to avoid cutting and patching.

  - BRACING:

Provide bracing and scaffolding necessary for masonry work. Design bracing to resist wind pressure as required by local building codes.

  - SEISMIC REQUIREMENT:

In addition, the Contractor shall provide additional seismic reinforcement. The total minimum reinforcing percentage for structural walls shall be 0.20 percent and non-structural walls shall be 0.15 percent. Bond beams are required at the top of footings, at the bottom and top of openings at roof and floor levels, and at the top of parapet walls.

# PART 2 – PRODUCTS

2.1 	MORTAR

2.1.1.  	Mortar Properties: Mortar for masonry in dosage per 1 m³ shall be realised of:

      - Lime mortar type 15 with river sand (which porosity of 40% and water content with relevant increasing of volume by 20%) mixed in proportion of cement: lime: sand = 1: 0, 8: 8. 110 lt hydrated lime, 150 kg cement (type 300), 1.29 m3 sand.

      - Lime mortar type 25 with river sand (which porosity of 40% and water connect with relevant increasing of volume by 20%) mixed in proportion of cement: lime: sand = 1: 0,5: 5,5. 92 lt hydrated lime, 212 kg cement (type 300), 1,22 m3 sand.

      - Lime mortar type 15 with clean sharp sand (to have a porosity of 35%) mixed in proportion of cement: lime: sand = 1: 0,8: 8. 105 lt hydrated lime, 144 kg cement (type 300), 1,03 m3 sand.

      - Lime mortar type 25 with river sand (to have a porosity of 35%) mixed in proportion of cement: lime: sand = 1: 0,5:5,5. 87 lt hydrated lime, 206 kg cement (type 300), 1,01 m3 sand.

      - Lime mortar type 1:2 with clean sharp sand (to have a porosity of 35%) mixed in proportion of cement: sand = 1:2. 527 kg cement (type 400), 0,89 m3 sand.

      - Water: Clean, potable, and free from substances which could adversely affect the mortar.

2.3	GROUT: Slump between 200 and 280 mm. Provide minimum grout strength of 13,800 kPa 2000 psi in 28 days.

2.3.1	Admixtures: Do not use air-entrainment, anti-freeze, or chloride admixtures.

2.4 	MASONRY ACCESSORIES

2.4.1	Horizontal Joint Reinforcement:  DIN EN 845-3 Fabricate from cold drawn steel wire. Wire shall be hot-dipped galvanized after fabrication. Reinforcement shall be truss type with two or more longitudinal wires welded to a continuous diagonal cross wire, or ladder type with perpendicular cross wires not more than 400 mm. Provide flat sections 3 m long, and preformed corners and tees approximately 800 mm long. Overall width shall be approximately 50 mm less than nominal thickness of wall.

2.4.2 	Anchors and Wall Ties: Provide approved designs of stainless steel,.

Dovetail, flat bar or wire anchors: flat bar: sheet steel, not lighter than 16 gage, and 22 mm, with end turned up 6 mm. Wire: not lighter than 6 gage, 22 mm with wire looped and closed.

2.4.3 	Reinforcing Bars: Provide positioners that prevent displacement of reinforcing during construction.

2.5 	SOURCE QUALITY CONTROL

    - Efflorescence Test: Conduct efflorescence tests on masonry units that are to be exposed to weathering. Schedule tests far enough in advance of starting masonry work to permit retesting if necessary. Submit to the Supervisor or his representative all data sheets for materials used building the masonry units.

# PART 3 - EXECUTION

3.1	PREPARATION

Prior to start of work, masonry inspector shall verify the applicable conditions as set.

3.1.1 	Protection:

- Stains: Protect exposed surfaces from mortar and other stains. When mortar joints are tooled, remove mortar from exposed surface with fibre brushes and wooden paddles. Protect base of walls from splash stains by covering adjacent ground with sand, sawdust, or polyethylene.

- Loads: Do not apply uniform loads for at least 12 hours or concentrated loads for at least 72 hours after masonry is constructed.

- Provide temporary bracing as required.

3.1.2	Surface Preparation: Surfaces on which masonry is to be placed shall be smooth, clean, and free of foreign substances when mortar is applied.

3.2	FIELD QUALITY CONTROL

3.2.1	Mortar Strength and Properties: DIN EN 1015-11, for the first 3 consecutive days, and each third day thereafter.

3.3	WORKMANSHIP: Carry masonry up level and plumb. Furnish and use story poles or gage rods throughout the work. Changes in coursing or bonding after the work is started will not be permitted. Step back unfinished work for joining with new work. Check heights of masonry at each starting and finis points to maintain the level. Drilling, cutting, fitting, and patching to accommodate the work of others shall be performed by masonry mechanics. Cut masonry with masonry saws for exposed work. Structural steelwork, bolts, anchors, inserts, plugs, ties, lintels, and miscellaneous metalwork specified elsewhere shall be placed in position as the work progresses. Provide chases of approved dimensions for pipes and other purposes where indicated and where necessary.

Cover tops of exposed walls and partitions not being worked on with a waterproof membrane secured in place and extended down at least 600 mm on both sides.

  - MORTAR MIXING:

Measure mortar materials in 0,0283 m3 containers to maintain control and accuracy of proportions. Do not measure materials with shovels.

Do not load mixer beyond its rated capacity. Do not use mortar which has stiffened because of evaporation by adding water and mixing to obtain a workable consistency. Do not use mortar which has not been placed in final position within 2 1/2 hours after the initial mixing.

Do not use antifreeze compounds, salts, or other substances to lower the freezing point of mortar.

  - Mortar: Mix mortar in accordance with DIN EN 998-2 to obtain type mortar required. When masonry cement is provided, conform to masonry cement manufacturer's printed mixing instructions.

  - Grout: Provide fine grout in grout spaces less than 50 mm  in any horizontal dimension or in which clearance between reinforcing and masonry is less than 20 mm.

  - MORTAR JOINTS:

Use uniform thickness of 10 mm for brick walls and 10 mm to 20 mm for stone walls, unless otherwise indicated. Tool exposed joints slightly concave with a round or other suitable jointer when the mortar is thumbprint hard. Horizontal joints shall be level; vertical joints shall be plumb and in alignment from top to bottom of wall within a tolerance of plus or minus 10 mm in 10 m.

3.6	TOLERANCES

Masonry work shall be within the following limits:

  - Face of stone : 1to 3 mm from face of adjacent stone

  - Face of Concrete Masonry Unit: 2 mm from face of adjacent unit.

  - Variation in Thickness: Plus or minus 6 mm.

  - BONDING AND ANCHORING:

Unless indicated otherwise, extend partitions from the floor to the bottom of the construction above. Structurally bond or anchor walls and partitions to each other. Securely anchor non-load-bearing external walls and interior partitions to the concrete structures in a manner that provides lateral stability while permitting unrestricted deflection of construction above. Completely embed anchors in mortar joints.

3.8 	CLEANING

3.8.1 	Protection: During cleaning operations, protect work which may be damaged, stained, or discoloured.

3.8.2 	Pointing: Upon completion of masonry work and before cleaning, cut out defective mortar joints and tuck point joints and all holes solidly with pre-hydrated mortar.

3.8.3 	Cleaning: Clean exposed masonry surfaces with clear water and stiff fibre brushes and rinse with clear water. Where stains, mortar, or other soil remain, continue scrubbing with warm water and detergent. Immediately after cleaning each area, rinse thoroughly with clear water. Do not use caustic solutions or sandblasting to clean surfaces. Masonry shall be free of stains, efflorescence, mortar or grout droppings, and debris. Restore damaged, stained, and discoloured work to original condition or provide new work.

# 6.  PLASTER FINISH

# PART 1 - GENERAL

1.1	REFERENCES: The publications listed below form a part of this specification to the extent referenced. The publications are referred to in the text by the basic designation only.

1.1.1	Applicable Serbian laws and norms

1.1.2	European Norms and Standards:

DIN 18350	Construction contract procedures (VOB) - Part C: General technical specifications in construction contracts (ATV); Plastering and rendering

DIN 18550-3	Plastering and rendering - Thermal insulating rendering systems made of mortar with mineral binder and using expanded polystyrene (EPS) as aggregate

DIN V 18559 	Composite systems for thermal insulation; concepts, general description

DIN 4102	Fire behaviour of building materials and elements - Classification of building materials - Requirements and testing

1.2	SUBMITTALS

Submit the following:

Samples

- Plaster: Submit four 900 mm panels of varying texture for the Supervisor's approval. After selection of an acceptable texture, construct a sample. The sample wall shall show all aspects of stucco work, including but not limited to, expansion joints, control joints, corner extrusions. The Contractor shall protect the sample wall from damage during the length of the contract.

- Natural stone cladding

Manufacturer's Instructions

- Submit manufacturer's printed mixing instructions for ready-mix plaster and stucco and acoustical plaster finish.

1.3	DELIVERY, STORAGE, AND HANDLING:

Deliver manufactured materials in the manufacturers' original unbroken packages or containers which are labelled plainly with the manufacturers' names and brands. Keep cementations materials dry and stored off the ground, under cover, and away from sweating walls and other damp surfaces until ready for use.

1.4	ENVIRONMENTAL CONDITIONS

1.4.1 	Cement Plaster and Stucco: Maintain an ambient temperature of not less than 5 degrees C continuously where cement plaster and stucco work will be performed.

      - Protection from Sun and Dry Winds: During the application of the finish coat, and for a period of 48 hours following the completion of finish coat application for any given area, protect the surface of the cement plaster and stucco from direct sunlight and direct winds. Use of tarpaulins or other temporary means are acceptable.

# PART 2 - PRODUCTS

MATERIALS

2.1	PLASTER

2.1.1	Product and Material: Conform to the specifications, standards, and requirements specified herein. Provide asbestos-free materials. Generally plaster shall be in accordance with DIN 18550-2

2.1.1.1	Cement plaster base coat: Cement Plaster Base Coat

2.1.1.2	Cement plaster finish coat: Cement Plaster Finish Coat

2.1.1.3	Hydrated lime

2.1.1.4 	Aggregates

Sand for Cement Plaster

- Sand Gradation: maximum 2 mm grain.

2.1.1.5	Water: Suitable for domestic consumption, and free of mineral and organic substances that affect the hardening and durability of the plaster or stucco.

2.1.2	Proportioning: Unless specified otherwise, materials are specified on a volume basis and shall be measured in approved containers, to ensure that the specified proportions will be controlled and accurately maintained during the progress of the work. . Prepare ready-mix cement plaster and for use by the addition of water only.

2.1.2.1 	Cement Plaster Base Coat

2.1.2.1.1 	Portland Cement Base Coat Plaster: Base coat types are two coats applied prior to the application of the finish coat. The two base coat types consist of the scratch coat and the brown coat. Mix scratch coat in proportion of one part by volume Portland cement, 0 to 3/4 parts by volume hydrated lime and 2 1/2 to 4 parts sand. Mix brown coat in proportion of one part by volume Portland cement, 0 to 3/4 parts by volume hydrated lime and 3 to 5 parts sand (volume of sand per sum of cement and lime).

2.1.2.2	Cement Plaster Finish Coat: Mix finish coat in the proportion of one part by volume of Portland cement to not more than one part by volume of hydrated lime, and not more than 4 parts by volume of damp loose sand. Workability shall govern the actual amount of lime and sand used in the finish coat, within the limits specified herein.

2.1.3	Colour: Colour of finish coat shall be approved colouring compounds to produce the required colour.

2.1.4	Mixing

2.1.4.1	Job-Mixed Materials: Mix materials in mixers except finish coats containing lime may be hand mixed. The mechanical mixers shall be an approved type that accurately and uniformly controls the quantity of water. When mixing by hand, mix dry plaster aggregate to a uniform colour in the mixing box, add water, and hoe the plaster immediately into the water and mix thoroughly to a proper consistency.

2.1.4.2	Ready-Mixed Packaged Materials: Mix ready-mixed packaged gypsum plaster cement plaster in accordance with manufacturer's printed instructions.

2.2	NATURAL STONE CLADDING

2.2.1 	Natural stone slabs:

The characteristics shall comply with the particular technical specifications described for reconstruction of amphitheatre in city park Bubanj.

2.2.2	Metal anchors (support cleats restrain tie)

Use as metal anchors for support and restrain the slabs support-cleats and restrain-tie in stainless steel 1.4571/1, 4401 (A4).

# PART 3 - EXECUTION

3.1	PLASTER

3.1.1	Surface Preparation: Clean surfaces before application from dust, loose particles, grease, bond breakers, and foreign matter. Do not apply plaster directly to surfaces of masonry or concrete that have been coated with bituminous compound or other waterproofing agents, or that have been painted or previously plastered. Before plaster work is started, wet masonry and concrete surfaces thoroughly with a fine spray of clean water to produce a uniformly moist condition. Check metal grounds, corner beads, screeds, and other accessories carefully for alignment before starting work. Cover all adjacent permanent works which may be soiled during plaster works.

3.2	WORKMANSHIP

3.2.1	Slump Tests: Apply Plaster by hand or machine. When a plastering machine is used, control the fluidity of plaster to have a slump of not more than 75 mm when tested, cement plaster.

3.2.2.	Application: Apply cement plaster, in three coats (two bases and one finish).

Plaster shall not be continuous across expansion and control joints occurring in walls, partitions, and ceilings. Finish work level, plumb, square, and true, within a tolerance of 3 mm in 2500 mm, without waves, cracks, blisters, pits, crazing, discolouration, projections, or other imperfections. Form plaster work carefully around angles and contours, and well-up to screeds. Take special care to prevent sagging and consequent dropping of applications. There shall be no visible junction marks in finish coat where one day's work adjoins another.

3.2.3	Cement and Stucco Plaster: Provide moist curing to the base coat until the application of the   finish coat. Immediately before finish coat application, moisten the base coat. For hot, dry, and windy conditions, moisten surface and cover with polyethylene plastic (weighted or taped down) to prevent water loss through evaporation.

3.3	CEMENT PLASTER WORK: Use cement plaster for exterior surfaces which will be subjected to abrasive action or frequent wetting and for exterior surfaces.

3.3.1	Cement Plaster Thickness Requirements

- Vertical Surfaces

Total Thickness is 13 mm to mm 15 mm

3.3.2 	Cement Plaster Base Coat Work: Apply the scratch coat to cover the base with sufficient material and pressure to form a good bond on the wall and ceiling base. Rake or score horizontally. Apply the brown coat after the scratch coat has been aged at least 24 hours in addition to the moist curing period. Apply the brown coat and float plaster surface to a true, even plane with rod or straightedge to receive the finish coat.

3.3.3 	Cement Plaster Finish Coat Work: After the brown coat has been moist cured for not less than 24 hours and aged at least an additional 5 days, apply the finish coat to a thickness of not less than 3 mm.  Where previous coat has become dry, dampen the surface evenly with water, prior to the application of the finish coat.

# 7.  PAINTS AND COATINGS

# PART 1 - GENERAL

1.1	REFERENCES

The publications listed below form a part of this specification to the extent referenced.

1.1.1	Applicable Serbian Laws and Norms

1.1.2	European Norms and Standards:

DIN 18363	Construction contract procedures (VOB) - Part C: General technical specifications in construction contracts (ATV); Painting and varnishing

DIN 18364	Construction contract procedures (VOB) - Part C: General technical specifications in construction contracts (ATV); Corrosion protection of steel and aluminium structures

DIN 18451	Construction contract procedures (VOB) - Part C: General technical specifications in construction contracts (ATV); Scaffolding work

DIN 4102	Fire behaviour of building materials and elements - Classification of building materials - Requirements and testing

DIN 6173	Colour matching General Rules

DIN EN 927-1	Coating materials and coating systems for exterior wood surfaces - Part 1: Classification and selection

DIN 971-1		Paints and varnishes - Terms and definitions for coating materials - Part 1: General terms

1.2	SUBMITTALS

Submit the following: Shop Drawings, Product Data For each type of coating, sealant, or other product furnished, submit data from the manufacturer's paint laboratory indicating that the product conforms to requirements of the referenced Samples Manufacturer's Instructions Certificates

1.3	QUALITY ASSURANCE

1.3.1 	Field Samples and Tests: The Client reserves the right to take 0.5 litre samples of paints at random from the products delivered to the site and test them to verify that the products either conform to the referenced specifications or the approved substitution. Products which do not conform shall be removed from site and replaced with new products that conform to the referenced specification or the approved substitution.

  - PACKAGING, LABELING, AND STORAGE:

Paints shall be in sealed containers that legibly show the contract specification number, designation name, formula or specification number, batch number, colour, quantity, date of manufacture, manufacturer's formulation number, manufacturer's directions including any warnings and special precautions, and name and address of manufacturer.

1.5	ENVIRONMENTAL CONDITIONS

1.5.1	Exterior Coatings: Do not apply coating to surfaces during foggy, rainy weather or strong direct sunlight. The creation of shading using tarpaulins etc. shall be permitted only with the permission of the Supervisor .

# PART 2 - PRODUCTS

2.1	MATERIALS

2.1.1	Paint and coating systems shall be complete so that all primers, thinners toners etc recommended by the manufacturer are included in order to ensure a complete system.

2.1.2	Colours shall be taken from the Project Particular Specifications

# PART 3 - EXECUTION

3.1	SURFACE PREPARATION: Remove dirt, splinters, loose particles, grease, oil, and other substances deleterious to coating performance. The underground shall be prepared to ensure the final quality of the coating is met. Small defects in plaster in wet rooms shall only be filled with non-gypsum based materials. In cases where the underground is considered unsuitable the Supervisor or his representative shall be immediately notified.

3.1.1 	 Preparation of metal surfaces

3.1.1.1	Ferrous Surfaces: Brush-off blast and clean entire surface. Water jetting to may be used to remove loose coating and other loose materials.

Galvanized Surfaces with Only Dirt and Zinc Oxidation Products: Clean with solvent, steam, or non-alkaline detergent solution.

3.1.1.2	Aluminium, Other Non-Galvanized and Non-Ferrous Surfaces: Surface Cleaning: Solvent clean in accordance and wash with mild non-alkaline detergent to remove dirt and water soluble contaminants. Surfaces shall be roughened.

3.1.2 	Preparation of concrete and cementations surface

      - Concrete and Masonry: Remove the following deleterious substances: Dirt, Grease, and Oil. Wash surfaces. Plastered surfaces shall be brushed using a copper brush prior to painting. Plaster imperfections shall be removed prior to painting.

    - Preparation of wood surfaces: Initial surface cleaning and sanding and scraping. Holes and depressions shall be filled with suitable filler. Where paint stripping spirits are used then the wood shall be neutralised prior to painting. Wood must be dry prior to painting. Sharp edges shall be rounded to a radius of approx. 2mm.

    - Façade Cleaning: The Contractor shall carryout a test clean for approval by the Supervisor or his representative prior to commencing with the cleaning works. Permanent works shall be suitable protected during cleaning works.

3.2	APPLICATION

3.2.1	Coating Application: Apply coating materials in accordance with manufacturer’s instructions. Thoroughly work coating materials into joints, crevices, and open spaces. Touch up damaged coatings before applying subsequent coats.

a. Drying Time: Allow time between coats, as recommended by the coating manufacturer, to permit thorough drying, but not to present topcoat adhesion problems. Provide each coat in specified condition to receive next coat.

b. Primers and Intermediate Coats: Do not allow primers or intermediate coats to dry more than 30 days, or longer than recommended by manufacturer, before applying subsequent coats. Follow manufacturer's recommendations for surface preparation if primers or intermediate coats are allowed to dry longer than recommended by manufacturers of subsequent coatings. Each coat shall cover surface of preceding coat or surface completely and there shall be a visually perceptible difference in shades of successive coats.

c. Finished Surfaces: Provide finished surfaces free from runs, drops, ridges, waves, laps, brush marks, and variations in colours.

3.2.2	Equipment: Apply coatings with approved brushes, approved rollers, or approved spray equipment, unless specified otherwise. Spray areas made inaccessible to brushing by items such as ducts and other equipment.

3.2.3	Thinning of Paints: Reduce paints to proper consistency by adding fresh paint, except when thinning is mandatory for the type of paint being used. Obtain written permission from the Supervisor or his representative to use thinners. The written permission shall include quantities and types of thinners to use.

3.2.4	Coating Systems:

a. Systems by Substrates: Apply coatings that conform to the respective specifications listed in the following points:

1 	Exterior Metal Surfaces

2 	Exterior Concrete, Concrete Masonry, Plaster

3 	Exterior Wood Surfaces

Minimum Dry Film Thickness (DFT): Apply paints, primers, varnishes, enamels, undercoats, and other coatings to a minimum dry film thickness of 0.0375 mm each coat unless specified otherwise. Coating thickness where specified, refers to the minimum dry film thickness.

- Utilised Product List: The Contractor shall submit a detailed list of all coatings and paints used following completion of the work. This list shall include use, manufacturer, product reference, supplier & charge number of the materials to enable the Client to re-order materials for maintenance purposes.

- Reserve for touch-ups: The contractor shall keep a reserve of each coating used in original colour to permit touching up of small damages which may occur before handing over the building.

3.3	COATING SYSTEMS FOR METAL

The Supervisor or his representative may request that multiple coatings of corrosion protection coatings are applied in different tones to enable the verification of the number of coats.

EXTERIOR METAL SURFACES

Rust proofing: 0.50 mm

Intermediate: semi gloss 0.0375 mm

Topcoat: semi gloss0.0375 mm

3.4	COATING SYSTEMS FOR CONCRETE AND CEMENTITIOUS SUBSTRATES

EXTERIOR

Primer: 0.0375 mm

Intermediate: (flat) 0.0375 mm

Topcoat: (flat) 0.0375 mm

3.5	PROTECTION

    - Protection of Permanent Works: The Contractor shall cover and mask all items in the area of the works to protect them from being soiled by paint. Any soiling shall be immediately removed. All areas painted shall be protected.

    - Storage of flammable materials: The storage of flammable materials on site shall be discussed agreed with the Supervisor or his representative.

    - Protection of Glass and Aluminium: When using lime based paints glass and aluminium shall be carefully protected, such as glass bridges, balustrades etc.

    - Protection of Seals: All joint seals to the glass covers or stone joints shall be protected from the paint. Where this is not practical then the seals shall be protected using masking tape.

    - Removal of surplus material: All surplus materials shall be removed from site unless otherwise stipulated by the Supervisor or his representative.

    - Early coating of concealed parts: Items which will be concealed by advancing construction which however require coatings shall be painted prior to their concealment. The Contractor is prepared to coordinate such work with the Supervisor or his representative even if it means early or interrupted presence on site.

    - Masking Tape: Masking tape shall be suitable for the surface upon which it shall mask. Material reactions shall be avoided. If in doubt tests shall be made on non visible parts of the surface.

    - Unsuitable surfaces: The following surfaces are considered unsuitable for painting and shall be masked during painting works:

- Foams

- Electrometric seals

- Polyamide, Soft PVC, Polyvinylchloride

- Polycarbonates

- Polyethylene

# 9.  PAVING WORKS

# PART 1 – PATHEWAY WORKS GENERAL

  - REFERENCES:

The publications listed below form a part of this specification to the extent referenced.  The publications are referred to in the text by the basic designation only.

1.1.1	Applicable Serbian Laws and Norms

    - European Norms and Standards:

DIN 18315	Contract procedures for building works - Part C: General technical specifications for building works; Construction works for traffic lines, top layers without binders

DIN 18316	Contract procedures for building works - Part C: General technical specifications for building works; Construction works for traffic lines, top layers with hydraulic binders

DIN 18318	Contract procedures for building works - Part C: General technical specifications for building works; Construction works for traffic lines, stone and tile pavements, kerbs

DIN EN 1338	Concrete paving blocks - Requirements and test methods.

DIN EN 1339	Concrete paving flags - Requirements and test methods.

DIN EN 1340-1341 Concrete kerb units; Requirements and test methods.

DIN EN 1342	Sets of natural stone for external paving - Requirements and test methods

DIN EN 1343	Kerbs of natural stone for external paving - Requirements and test methods

EN 12440         Natural stone- Denomination criteria

EN 12670         Natural stone- Terminology

EN 1925, 1926, 1936, 12370, 12372, 12407, 12371, 13161, 13364, 13755, 13919, 14231, 13373, 14066, 14147, 14205, 14146, 14158, 14157, 14579-Europian standards on natural stone test methods.

EN 13474-1-2-3 for Structural Glass Pavements

  - GENERAL REQUIREMENTS:

The paving shall consist of full depth pavement section as shown on the drawings or application over base in compacted mixed grade gravel with concrete sub base and a steel mesh. Place the pavement and sidewalk as indicated on drawings. The characteristics of the stone pavements mast fulfil all technical requirements as specified under the section of “Thermal and environmental rehabilitation of open spaces”.

The technical behaviour of the stone tile should be tested at the professional laboratory and all data received must be submitted for approval to the Supervisor or his representative, prior starting the work.

    - Pre-construction Conference:

A joint working conference attended by the Supervisor or his representative, the General Contractor, and any sub-contractors shall be held prior to actual paving operation start to confirm the standards required, drainage slopes, traffic flow and control, paving work sequencing, work start and stoppage, requirements for safety signs and barricades and other construction constraints that need to be considered as well as the paving samples and colour approved by the Supervisor or his representative.

1.2.2	Walk-Thru: Prior to actual paving operations, a walk-thru of the entire project area by the Supervisor or his representative and Contractor shall be conducted. The Contractor must have the permission by writing by the Supervisor or his representative to start paving work and to ensure the following:

- All levels have to be checked prior commencing the work. No obstructions or stalled vehicles will impede paving operations;

- All compaction requests have been satisfied;

- No obstructions or stalled vehicles will impede paving operations;

- Required surface preparation has been accomplished and area is ready for execution of sub base

  - DEFINITION OF TERMS:

- Sub grade: Existing or borrow earth foundation upon which pavement is constructed

- Compacted Gravel Sub base: for road Course depth as show on the drawings of mixed grade ruled gravel between the sub grade and the aggregate base course

- Concrete Sub Base Course: depth as show on the drawings which is above the Gravel Sub Base layer depth 20 cm and cast in situ concrete depth 20cm reinforced with welded or tie steel wire 20x20 cm  diameter 8mm and for the side walk is cast in situ concrete depth 10 cm reinforced with welded or tie steel wire 20x20 cm T 131 steel mesh

- Paving Stone Base; as specified on the drawings, the dimensions of the tiles are 10x15x5 cm, colour is light grey or light beige.

- Side walk pavement, the dimensions of the tiles is 20x20x5, and the colour of the tiles must be same as the path or light different colour from the tiles of the path.

- Blind corridor tiles are the same tiles as the path, with dimension 40x40x5 cm

- Wearing course for side walk depth as indicated on drawings.

- Wood Pathways

1.4	SUBMITTALS

    - Samples: (a) submit samples of materials only as directed by the Supervisor or his representative, (b) in lieu of actual samples provide certificates of Compliance for each of the materials listed below with approval by Supervisor or his representative:

- Granular mixtures

- Concrete samples

- Steel mesh samples

- Paving stone samples

- Samples for blind paving stone

- Paving wood samples

- Materials for curing concrete

- Type of bolts to be used

- One sample of hot deep galvanise structural steel

- Pavement marking paint.

1.4.2	Certificates: Submit to Supervisor or his representative all quality certificates and technical descriptions of all materials to be used

1.4.3	Test Reports: Before delivery of materials, certified copies in triplicate of the reports of all tests required herein under materials and in referenced publications shall be submitted to the Supervisor or his representative for approval. Additional testing shall be submitted when the source of materials is changed.

Submit certified test reports for the following:

- Granular mixtures

- Stabilised granular mixtures

- Stone tiles

- Steel mesh

- Concrete

  - DELIVERY AND STORAGE:

1.5.1	Delivery and storage of materials for concrete for sidewalk and base (except ready-mix concrete): Packaged materials shall be delivered to the project site in their original, unopened package or container bearing manufacturer's label clearly identifying manufacturer's name, brand name, material, weight or volume, and other pertinent information and stored in their original, unbroken package or container in a weather tight and dry place, until ready for use. Unpackaged aggregates shall be stored in a manner as to avoid excessive segregation or contamination with other materials or other sizes of aggregates. Store reinforcements of different sizes and shapes, in separate piles or racks raised above the ground to avoid rusting. Protect from contaminants such as grease, oil, and dirt. Provide for accurate identification after bundles are broken and tags removed.

1.5.2	Paint marking materials shall be delivered and stored in sealed containers that plainly show the designated name, formula or specification number, batch number, colour, date of manufacture, manufacturer's name, and directions, all of which shall be plainly legible at time of use.

  - TRAFFIC CONTROL:

Vehicular and utility vehicle traffic shall not be permitted on the pavement until the Supervisor or his representative has given the permission by writing to the Contractor. Place barricades and warning signs prominently near the beginning of the work site and around access points to the project site area for alerting vehicular and pedestrian traffic of construction operations and/or obstructions.

The use of bunting or multi-coloured, striped cloth materials that can easily be blown by the wind, is not allowed as barricade.  Use barricades and warning signs that will withstand the weather and perform their intended function even after hours of darkness.

1.7	WEATHER LIMITATIONS:

1.7.1	Levelling, Binder and Wearing Courses: Place the gravel mixture, concrete, steel mesh, wood pavements and paving stone only during dry weather and on dry surfaces. Place the paving stone only when the surface temperature of the underlying course is greater than 5 degrees Centigrade.

1.8	SAMPLING AND TESTING:

1.8.1	The Contractor shall conduct required tests on the sub base and base mixtures also for conformance with specified acceptance quality level

    - The Contractor shall conduct required tests on the concrete sub base for conformance with the technical specification.   No payments shall be made to the Contractor for pavements or portions of pavements removed for non-conformance and replaced with new pavements, which meet the requirements of this specification.

# PART 2 - PRODUCTS

  - WATER: Shall be soft, potable and free from any suspended inorganic material and chloride sulphate compounds exceeding the percentages indicated. Water shall be slightly alkaline with PH between 6 and 8, shall not include more than 2 parts per 100, by weight, of material in suspension and not more than 15 parts per 100, by weight, of material in solution. It shall not contain organic matter either in suspension or in solution. Sulphate shall not be more than 1 part per 1000 by weight. Sea water is prohibited.

  - GRANULAR MIXTURE

2.2.1	For the construction of the sub base of the road it will be necessary to use a suitable mixture of granular materials, starting with clay silt 0.074mm and up to gravel and crushed stones with a maximum dimension of 50mm. The thickness of the stabilised layer will be determined in relation to the load bearing sub grade and of the loads that must be supported by the traffic decided on the technical specification.

  - CONCRTE SUB BASE

    - Acceptance quality level: Before starting the concrete sub base, a check has to be done for all Levels and a report has to be submitted by writing to the Supervisor or his representative. The Supervisor or his representative will give an instruction for starting the concrete sub base as shown on the drawings.

  - STONE LAYER: The stone layer has to be constructed as shown on the detail design. The stone dimensions are different in the different sites. Always, the Contractor shall supply to the Supervisor or his representative the samples of the stone, and should get approval by writing for the stone which will be used. The stone has to be light grey colour, or light beige colour. The structural bearing load for the stone pavements at the road area must be 420 kg/cm2 and over for paving on pedestrian and vehicular use. This test should be done in the accredited testing laboratory by the contractor and should be supplied to the Engineer prior commencing the work. CE marking shall be appeared in each stone packaging or accompanying with commercial documentation as per EU standards.

  - CONSTRUCTION EQUIPMENT:

Provide equipment dependable and adequate for the purpose intended and properly maintained in satisfactory and safe operating condition at all times. Calibrated equipment such as asphalt distributors, scales, batching equipment, spreaders and similar equipment, shall have been recalibrated by an approved calibration laboratory within 12 months prior to commencing work (and every 12 months thereafter, by such laboratory from the date of recalibration, during the term of the contract).

2.5.1	Motorised grader: they must be self-propelled, supplied with pneumatic tyres with a basic tyre width less than 4m.

2.5.2	Spraying equipment: composed of water distribution trucks using a bar to spray uniformly and in variable and controlled quantities.

2.5.3	 Soil compactor: composed of:

a)	Sheep foot roller: with simple or double cylinder adaptable to compact the indicated materials.

b)	Mechanical Tamper: supplied with rubber tyres pulled by a rubber wheeled tractor

c)	Hand Tampers: Minimum weight of 12 kilograms with a tamping face of not more than 300 square centimetres; shall be used in areas not accessible to rolling equipment.

d)	Vibrating roller: with the capacity to develop a variable static load between a min. 300Kg and 1,300Kg

- Compressor roller: shall have a minimum weight determined by Works Director. The wheels shall have adjustable scrapers, water tanks and sprinkling apparatus to keep the wheels wet and thus prevent the bituminous mixtures from sticking to the wheels. The rollers shall be capable of reversing without backlash and shall be free from worn parts. The roller's wheels shall not have flat or pitted areas or projections that leave marks in the pavement.

- Mechanical Distributor: adjustable and capable of distributing uniformly the quantified materials for every surface sq. m

- Mixing equipment: road scarify, wheel scraper etc.

# PART 3 - EXECUTION

  - APPLICATION OF SUBASE COURSE:

Granular material taken from a quarry, or crushed or from an alluvial bank, properly screened, must be spread in long strips on the path surface. To obtain a course thickness as indicated in the drawings successively, carry out the procedure of the mixing in order to obtain a good homogeneity using a motto grader and by laying at the same time on the road surface. Then it is convenient to dampen the material, proceed to compact the course with rubber or vibrating rollers reaching a density required.

The layer of material should conform according to the transversal and longitudinal profile of the project.

  - APPLICATION OF BASE COURSE:

For the procedure of application of the base course the work should be carried out as indicated in the technical specification and further:

- The concrete should be cast uniformly in the required quantities and the work should satisfy the Supervisor or his representative.

- Water should be added in the required quantity using pressure spraying bar and uniformly incorporated into the mixture in order to obtain the requested humidity by the Supervisor or his representative.

The proportion of cement in the mixture will be determined on the requirements of the standards.

  - EXECUTION OF  CONCRETE SIDEWALK SUB BASE AND ROAD SUB BASE:

3.3.1	Ready-mix Concrete Transporting: Ready mix concrete furnished in revolving concrete mixers on trucks will be acceptable provided the requirements of DIN EN 206 and the following are met:

3.3.2	Placing: Concrete shall be poured monolithically. Place concrete within 90 minutes of either addition of mixing water to cement and aggregates or addition of cement to aggregates if the air temperature is less than -30 degrees Centigrade.  Reduce mixing time to 60 minutes if the air temperature is greater than 30 degrees Centigrade. Additional water may be added, provided that both the specified maximum slump and water-cement ratio are not exceeded. Do not place concrete when: (a) weather conditions prevent proper placement and consolidation; (b) in uncovered areas during periods of precipitation; and (c) in standing water. Prior to placing concrete, remove dirt, construction debris, water, snow, and ice from within the forms.

3.3.2.1	Concrete Pumps: Concrete may be conveyed using concrete pumps only after written approval by the Supervisor or his representative. In requesting approval, the Contractor shall submit his entire plan of operation from time of discharge of concrete from the mixer to final placement in the forms. Concrete pumps shall be operated and maintained so that a continuous stream of concrete is delivered into the forms without air pockets, segregation, or change in slump exceeding 5 centimetres.

3.3.2.2	Concrete Vibration: Immediately after placing, each layer of concrete shall be vibrated using internal concrete vibrators supplemented by hand spading, rodding and tamping. Tamping or other external vibration of forms will not be permitted. Vibrators shall not be used to transport concrete inside forms. Internal vibrators submerged in concrete shall maintain a minimum frequency of not less than 8000 vibrations per minute.   Duration of vibration shall be limited to time necessary to produce satisfactory consolidation, generally from 5 to 15 seconds. Vibrators shall be applied at uniformly spaced points of 45 centimetres

3.3.3	Cold Weather: Provide and maintain 10 degrees Centigrade minimum concrete temperature. Do not place concrete when the ambient temperature is below 5 degrees Centigrade. Cover concrete and provide with a source of heat sufficient to maintain l0 degrees Centigrade minimum while curing.

3.3.4.	Hot Weather: Provide and maintain 32 degrees Centigrade maximum concrete temperature. Cool ingredients before mixing, or use other suitable means to control concrete temperature to prevent rapid drying of newly placed concrete. Shade the fresh concrete and start curing as soon as the surface of the fresh concrete is sufficiently hard to permit curing without damage.

3.3.5	Construction Joints: On driveways, with width of up to 4 meters, provide control joints with spacing not to exceed 4 meters and on the sidewalk shall be every 3 m by depending on the width of the sidewalks.

3.3.6	Surface Finishes:

3.3.6.1	Defects: Repair formed surfaces by removing defects. Exposed surfaces shall be uniform in appearance and, finished to a smooth form finish unless otherwise specified.

3.3.6.2	Floated Finish: Place, consolidate, and immediately strike off concrete to obtain proper contour, grade, and elevation before bleed water appears. Permit concrete to attain a set sufficient for floating and supporting the weight of the finisher and equipment. Do not spread dry cement over slab surface to absorb bleed water. Do not introduce a "topping mix" over the floated finish of a slab to achieve a smooth finish. Steel towelling shall be done over a floated finish before fresh concrete sets. Surface shall be level to within 6 millimetres in 3 metres where floor drains are not provided.

3.4	FIELD QUALITY CONTROL:

3.4.1	Tests: Perform the following minimum number of tests to ensure compliance with the temperature, thickness, and density requirements of all of the courses. Furnish all tools, labour and material for samples and for satisfactory replacement of pavement. Take samples and test at not less than the frequency required for each day’s work as a minimum; and at each change in the mix or equipment. Perform the following tests:

3.4.1.1	Road Sub base and Road Base In-place Density Test: ASTM D1556 or D2922.

3.4.1.2	Thickness Test: Determine thickness of sub base, base, binder and wearing course. Remove samples for the determination of thickness of the completed course in the presence of the Supervisor or his representative. One test has to be done for each 50 m3 concrete.  The maximum allowable deficiency at any point shall not exceed 10 millimetres of the required thickness. The Contractor shall correct areas found deficient by removing the pavement in question and replacing with new pavement.

3.4.2	Test Results of each test shall be forwarded as part of "Daily Report of Inspector".

3.5	FINISHED GRADE:

The finish grades of each course placed shall not vary from the finish elevations, profiles and cross section indicated on the drawings by more that 2 millimetres. The Contractor shall correct deficient paved areas by removing existing work and replacing with new materials meeting the specifications without additional cost to the Client. Skin patching for correcting low areas will not be permitted.

  - PROTECTION:

Keep all traffic off surfaces which are freshly treated. Provide sufficient warning signs and barricades so that traffic will not travel over freshly treated surfaces. Protect the treated areas from the traffic.

# WORKLIST ITEMS

| # Part 1 | # part 1 MOBILIZATION AND DEMOBILIZATION The price of this part of the work includes mobilization and demobilization of the resources including the accommodation for the supervisor, site information board. The Office for the Supervisor shall have enough space to accommodate the Supervisor and his representative for the whole length of the project. All the necessary facilities (power, phone lines, internet connection, and water and air condition) should be ensured. Two site information boards shall be fixed at visible places along the main road. Size and content of information shall be agreed with the Supervisor. It is necessary to provide the visibility of the project and to provide on board the name of the project and the co-financing institutions written in a way /text and font size/ conforming to the requirements of the Contract and in compliance with EU visibility guidelines and the guidelines by the Bulgaria-Serbia IPA Cross-border programme. The cost of these boards will be included in the Contractor’s rate in Bill of Quantities | lump sum |
| --- | --- | --- |
| # Part 2 | # Part 2 excavation work 2.1 Cleaning, cutting of shrubs and trees Cleaning, cutting with hands of shrubs and trees of 15 cm in diameter at the eastern side of Memorial park towards the residential street. Shrubs and trees to cut at the level of the ground put on truck and take to waste dump that is determined by investor- distance up to 10 km. Calculation to make per m2 all together for 50000.00 m2 Prior commencing the work, the Contractor shall notify well in advance the Supervisor or his representative on the beginning of any excavation for structures, so that the Supervisor or his representative may observe the cross-sectional elevations and measurements taken of the existing ground, and structure. Materials which cannot be removed from the site daily shall be appropriately stored in areas designated by the Supervisor or his representative. The Contractor shall provide suitable containers for the disposal of miscellaneous construction debris and shall empty these as often as required. The Contractor shall produce evidence of the proper disposal of materials upon request of the Supervisor or his representative. After finishing the excavations, the Contractor and the Supervisor or his representative shall make a detailed inspection of all adjacent levels and findings in order to determine its pre construction condition. 2.2 Transport of debris and disposal_________________________________________ Price includes transporting all the material resulting from excavation work, except as indicated or specified otherwise, removing from the site and deposited at the location given by the Municipality, up to 10km from the site. No cost will be added in case of changing the location from the municipality for depositing the excavated material during the project execution. | lump sum lump sum |
| # Part 3 | - Sanitation of pedestrian path - Sanitation of pedestrian path of 200 cm width around `auditorium` and from auditorium to pyramid monument at southern side of complex paving with stone (granite) blocks. For projected trace at first remove upper humus layer of ground in height of 30 cm, and then to put tampon layer of gravel in the height of 10-15 cm. From the edge of digging place (trace) to make two rows of stone blocks put on the foundation of concrete MB20 that will have the function of edging spot. Inner space to fill with smaller gravel in two layers up to the edge of the roadside. Dug soil to put at the surrounding terrain. In total 1200m2 should be made. - Sanitation of pedestrian paths of 150 cm of separated gravel. From projected trace first remove upper humus layer of ground in the height of 30 cm, and then put tampon layer of gravel in the height of 10-15 cm. Along the edge of dug (trace) to do two rows of stone blocks put on the layer of concrete МБ 20, that will have the function of pavement. Inner space to fill with smaller separated gravel in two layers up to the height of the pavement. Dug soil to put at the surrounding terrain. In total 2700m2 should be made. | lump sum lump sum |
| # Part 4 | - Making and setting of wooden resting places for sitting Making and setting of 15 wooden resting places for sitting (of pine wood). Resting places to be made in outer dimensions ~ 3 х 3м on wood pillars with ribs, horns, pliers and on way (or more ways) roof with covered boards. To make table (dim 90х240cm, made of pine wood) and to put benches in the middle part of this resting spot For every wooden pillar to make foundation –basis in the ground of 40х40 cm dimensions and depth of 40 cm. Into foundation bottoms to pour tampon of gravel in the layer of 10 m height, and to fill foundations with concrete МБ 20 and to put into them anchors from РА Ф22 to which wooden pillars will lean on over metal `shoe`. Dug soil to put at the surrounding terrain. | # lump sum |
| # Part 5 | - Sanitation and conservatory Restoration works on the object of pyramid type of monument made of congested stone. Sanitation means cleaning of all surfaces of erosion mortar from fugue joints, cleaning of lichen and moss and again putting fugue of joints with stronger mortar. If necessary certain unstable stone blocks make firmer (again building) with stronger mortar. Waste material to remove to garbage dump up to 10 km. Surface area is 48m2 | lump sum |
| # Part 6 | - Purchasing and setting of new marble memorial plaque-plate At the plaque of ~50х80х4cm of white marble to carve original text in mechanic way and plaque make firmer for the wall of the monument and polish afterwards. The letters should be painted in golden colour already determined for this sort of works. Calculation all together, purchasing of plate, writing and colouring of text, setting. | lump sum |
| # Part 7 | - Adaptation of plateau- down area, around monument, dimensions 6 х 6м Adaptation means removal of humus in the layer of 30 cm thickness, purchasing and filling of tampon with natural gravel of 20 cm of height, putting plates of granite stone blocks. Stone blocks to put in the layer of sand, and clutches of width of 1 cm to pour with cement milk. Three rim rows of granite blocks to put on the layer of concrete МБ 20. Along the rim/edge to put pillars – balls with stand (5 on every side, total 16) of castiron, connected with chains of castrion. Dug soil to put at the surrounding terrain. Area is 36m2. | lump sum |
| # Part 8 | - Making of seats over concrete stands of amphitheatre The seats to be made of wood (Christmas tree-abies) dimensions 8х6cm that are entrenched screwing. Joists should be processed, then process it with sandpaper and in final paint it with wood colour in tone that is determined by Institution. Area is 560m2 | lump sum |
| # Part 9 | - Removal of broken concrete footer of the seats Removal of broken concrete footer of the seats-934 items in amphitheatre in mechanical way (compressors). Seats are in the shape of `Г` letter with outer dimensions ~40х40х45cm, removal should be done with special care not to provoke damaging of healthy seats. | lump sum |
| # Part 10 | - Making and embedding of the footer of the seats in amphitheatre Seats-footer to be made of reinforced concrete МБ 30 in the form and dimensions of original seats and anchor it into concrete footer. Calculation per item all together; making, embedding, seats and foundations for 934 items | lump sum |
| # Part 11 | - Purchasing and setting of park benches 30 Park benches with concrete stand and wooden seats (without backrest) will be placed along pedestrian paths. Concrete stands to be anchored (embbed in the ground); for every bench to dig two trenches (holes) in the ground 30х60х40 cm, fill with tampon of in the layer of 10 cm, to concrete foundations and for them anchor concrete stands of benches. Dug soil to put all over the surrounding terrain. | lump sum |
| # Part 12 | - Reconstruction of present drink-fountain in the middle circle plateau in the eastern part of theatre First to remove humus layer in wide dug in the layer of 40 cm of height, and then put the tampon of river type gravel in the height of 10 cm. Tampon layer to be pushed mechanically (f.e. vibro plate…) and over it make the floor plate of plateau of drinking fountain of АБ type МБ 20 with sewerage installation. To cover drinking fountain from outer and upper side with mosaic. The body of fountain-bottom and sides (width 10 cm) to make of reinforced concrete МБ 20, and body of drinking fountain as special sculpture form. The space of down part, out of fountain and drinking fountain, to cover with granite stone blocks, clutches to fill with cement milk. Calculation for 70m2 of surface, all together, project elaboration, material purchase, making (sculptor) | lump sum |
| # Part 13 | - Leveling Leveling of soil surfaces, grounds, cleaning from stumps, forming good geometry around dyke around amphitheater Calculation for 700 m2 of surface taken for works (earthworks…) | lump sum |
| # Part 14 | - Reconstruction of two plateaus of diameter 20 m Cleaning of the terrain from shrubs. Taking off the layer of humus in the width of 20 cm, and wide dug for leveling of the terrain. Putting tampon layer of gravel of 10-20 cm. Putting layer of sand below blocks in the layer of 3-5 cm. Gravel is buldged with vibro bludge machine. Embedding present granite blocks 10 x 10 x 10 cm. Pour clutches with sand or cement-2 items | lump sum |
| # Part 15 | - Placing wooden bottoms on the present concrete rim Placing wooden bottoms on the present concrete rim. Seats to make of wooden fir tree material talp dimensions 8х6cm, and to fasten them with screwing. Ribs to refine with sandpaper, and in final color it with color in tone that is determined with the Institution. Calculation for 70m2 | lump sum |
| # Part 16 | - Cleaning central plateau from soil layers and grass Cleaning to do carefully so not to come to damaging of stone plateau plates. Excessive soil and grass to put all over the terrain. Preparation and recultivation of soil, as well as semination and forming lawns for in total 2000 m2. | lump sum |
| # Part 17 | - Adaptation of greens around amphitheatre Adaptation of greens around amphitheatre, through Forming of lawn, preparing soil, changes of grass seeds, digging and rolling with grass seedlings for 700m2, and Planting of softwood plants and deciduous in total 145 items | lump sum |
| # Part 18 | - Adaptation around path 1 and plateau from amphitheatre to old monument Forming lawn, selections of grass seeds, digging and rolling with grass planting for m2x3m, in total 1800. Planting of 300 softwood seedlings and planting of 80 decorative hedge | lump sum |
| # Part 19 | - Adaptation of greens along the path and around middle plateau 1B Creating lawns, preparation of soil, selection of grass seeds, digging and rolling with sowing grass for the area of 600 m2. Planting of deciduous seedlings in total 15 items. | lump sum |
| # Part 20 | - Adaptation of greens around plateu 1a Creating lawns, preparation of soil, selection of grass seeds, digging and rolling with sowing grass for1400 m2. Planting of deciduous seedlings in total 15 items | lump sum |